| Station |  | Task |  |
| --- | --- | --- | --- |
| 8 |  | 2 |  |
| Stationary V-810 G1 |  |  |  |
|  |  |  | ATV |

# Parts identification/installation – V-810

## Task Objective

At the completion of this task the technician will be able to properly identify the differences between similar engines, the differences between components from the same engine and the right installation of them.

# Materials

V810 engine disassembled to check parts

V500 piston

Vernier caliper

# Introduction

Proper identification of components will prevent the technician from making mistakes during assembly thus saving time and possible engine damages.

Even though, this engine is easy to assemble, there a couple of tips to keep in mind.

| - V-Twins Crankcase Oil Groove: |  |  |
| --- | --- | --- |
| When the V-500 engine was being developed it was found that there was a need for more oil on the skirt on the front piston. ROTAX remedied the situation by machining a groove in the crankcase at an oil channel to allow oil to be sprayed on the piston. Now, since all V-twins use the same crankcase, this groove will be found on all production engines and as well on replacement crankcases. Be sure that upon assembly sealant doesn't cover that groove up, as if it does, the groove can’t do its job. |  |  |
| - V-Twin Breather gear: |  |  |
| There are 2 executions of breather gears on V-twin engines. The running change started on MY 2008 models. The old breather gear 420 434 305 work with the PTO cover 420 611 146. The new breather gear 420 434 307 work with the PTO cover 420 612 125. |  |  |
| - Position of connecting rod-plain bearing |  |  |
| There is no difference on the edges of the plain bearings or connecting rods. NEW connecting rods can be installed in either way; only lower cap and rod must match together since there is a cracked surface. For USED connecting rods; running direction of the big end bearings and piston pins must not change. The notice "The plain bearing tapered end must be against the counterweight." is not correct on old shop manuals. |  |  |
| - V-Twin Engine Piston Installation: |  |  |
| V810: Since these pistons have an offset piston pin, both pistons have to be assembled with the marks pointing toward the same direction. At the end, both pistons should points to the rear side of the engine block. In other words, the mark on the top of the piston from the front cylinder (1) points toward the intake side, the rear cylinder (2) piston mark points the exhaust side. 1. Piston of cylinder 1 2. Mark on piston must show to intake side of cylinder 1 3. Piston of cylinder 2 4. Mark on piston must show to exhaust side of cylinder 2 V490 / V660: These pistons have NO offset of the piston pin and have to be assembled with the mark to the exhaust side to avoid contact between a piston and the valves. 1. Piston of cylinder 1 2. Mark on piston must show to exhaust side of cylinder 1 3. Piston of cylinder 2 4. Mark on piston must show to exhaust side of cylinder 2 |  |  |
| Measure the offset of the piston with the caliper |  |  |
| - New lonblocks |  |  |
| - The longblocks are assembled with the crankshaft with MAG side oil bore for 650W magneto cooling – CAPS is provided on ALL replacement engines even though MY2010 and newer 500 and 650 do not use this sensor. In this case, the CAPS is there to cover the opening, it does not need to be connected. - Use M12 bolts and washer to secure front and rear propeller shafts. |  |  |
| - Replacing Valve Guides |  |  |
| After installing a new guide it must be reamed with a commercially available 5mm reamer to ensure proper valve stem clearance. |  |  |
| - Cylinder heads |  |  |
| The cylinder heads are not identical in design. Do not invert the cylinder heads at assembly: If equipped, the CAPS sensor is located on the cylinder head #1 If there is no CAPS, note the chain tensioners are located on the rear side of the cylinders heads |  |  |
| - Camshaft |  |  |
| The engine is equipped with two different camshafts. |  |  |