| Station | Task |
| --- | --- |
| 12 | 2 |
| Stationary V800 Engine |  |

# 4-Stroke Engine Leak-Down Test

## Task Objective

At the completion of this task, the technician will be able to properly perform a 4-stroke engine leak-down test.  He will be able to give explanation and make clear the results he obtained so that he can determine the condition of the top end of the engine.

Materials

# Engine leak down test kit

# Crankshaft locking tool

# 2008 V-Twin Shop manual

Repair Order Information

The customer states there is a lack of engine power and the engine is running rough.

Introduction

Compression and Leak-Down Tests

In addition to a properly proportioned mixture of air and fuel and strong and properly timed ignition spark, the engine must produce enough compression to compress the air/fuel charge to a point where it is dense enough to be easily ignited.  Anything that reduces an engine’s ability to adequately compress the air/fuel mixture will negatively affect engine performance.

There are two methods by which an engine’s mechanical integrity can be tested; a cranking compression test (dynamic compression testing), and a cylinder leak-down test (static compression testing).

A cranking compression test is performed using a compression tester (Gauge) which threads into the sparkplug hole.  With the ignition disabled and the throttle wide open, the engine is then cranked, either manually with a kick or pull starter or with the engine’s electric starter, and a pressure reading is taken.  The test reading is then compared to the engine’s cranking compression specification to determine if the compression is within spec.

A cranking compression test will tell you if an engine is actually providing compression.  If compression is low or there is no compression at all, obviously there is a problem.  A cranking compression test will not, however, indicate a minor or slow loss of compression.  A cylinder leak-down test is much more effective in determining if a minor or slow compression leak exists.  A cylinder leak-down test is also performed to determine where the engine is losing compression.

# Compression Test

Compression Testing

In order to obtain accurate compression pressure results the following steps must be taken:

On electric start vehicles these tests must be performed with a fully charged and tested battery.--- Slow cranking speeds effect compression pressure.

On engines equipped with a decompressor lower than normal reading may be normal – comparison with a known good vehicle will give an indication of a normal reading

The ignition system must be disabled.---Ignition system damage may result.

Sparks can ignite fumes in the engine compartment.---Fuel injectors must be disabled.

Fuel vapors can be pumped into engine compartment.---

The compression gauge is inserted into the spark plug hole.---The engine is cranked over four to five times.

The reading from the gauge is recorded.

The readings are compared to specification.

Cylinder Leak- Down Test

A cylinder leak-down test requires a pressure gauge, a hose and fitting assembly, one end of which threads into the sparkplug hole, the other end connects to a source of compressed air.   The leak-down test tool also includes a regulator to control air pressure.

One side of the leak-down test tool is threaded into the engine’s spark plug hole.  The other end is attached to a compressed air source.  The engine is then brought to top dead center of its compression stroke, where both intake and exhaust valves are fully closed.  The crankshaft must then be rotated to exactly TDC, and in some cases secured so that it cannot rotate.

Note: A cylinder leak-down test can be performed on some engines with the piston at bottom dead center of the power stroke.  On most high performance 4-stroke engines, however, the exhaust valve is already beginning to open at BDC, so the crankshaft must be secured before BDC.

CAUTION: Refer to the engine’s shop manual for the recommended procedure(s) for securing the crankshaft and the maximum recommended leak-down test pressure.

Once the leak-down test tool has been installed on the air supply hose, the regulator is used to set the gauge needle to 0% of leakage.  The hose that is screwed into the cylinder is then connected to the regulator.  This allows compressed air to fill the engine’s cylinder to the specified pressure.  The gauge is then observed to determine the percentage of leakage of the cylinder.  The drop is compared to the allowable leakage as listed in the shop manual and repairs are made if appropriate.   A cylinder leak-down test can identify leaking intake or exhaust valves and worn compression rings.   If the percentage of leakage is excessive but there is no audible leak at either the intake or exhaust valves, the problem may be the piston’s compression rings. This can be further confirmed by injecting a small quantity of oil into the cylinder, which will help the rings provide a better seal.   If the percentage of leakage then goes down, the piston rings are likely worn beyond their service limit.  A leaking head gasket can also be at fault; removing the radiator cap and looking for bubbles inside will give a good indication if there is a leak into the cooling system.

Cylinder leak-down test

% Indicateur de fuite

LEAKAGE PERCENTAGE ENGINE CONDITION

0% to 15% Excellent condition

16% to 25% Good condition

26% to 40% Fair condition reduced engine performance

41% and higher Poor condition, diagnose and repair engine

# Cylinder Leak-Down Test

Refer to the Shop manual for the correct procedure and setting specifications.

# Procedures

|  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| - Lock crankshaft. |  |  |  |  |  |
| - Perform cylinder leak test as per service manual |  |  |  |  |  |
| - What is the percentage of leakage? |  |  |  |  |  |
| - Does this reading indicate a required engine repair? |  |  |  |  |  |
| - YES |  | NO |  |  |  |
| - If so what needs to be repaired? |  |  |  |  |  |

### Questions

1.  What have you learned from this task?

____________________________________________________________________________________________________________________________________________________________________________________________________________________________

2. A technician is checking a brand new engine with a leak test and discovers that the percentage of leakage is 20 percent.  Would this indicate that the engine is in poor condition?

YES

NO

3. Explain your answer to the preceding question.

____________________________________________________________________________________________________________________________________________________________________________________________________________________________

Instructor sign off--  Go __________