| Station |  | Task |  |
| --- | --- | --- | --- |
| 6 |  | 1 |  |
| Gearbox Shimming |  |  |  |
|  |  | ATV | Side-by-Side |

#### Output Shaft and Bevel Gear Shimming / Outlander V-Twin Gearbox

## Task Objective

At the completion of this task the technician will be able to properly disassemble, inspect and reassemble a gearbox for the Outlander V-Twin recreational vehicles.  The technician will be able to check the operation of the gearbox before and after disassembly, inspection and reassembly.  In addition, the technician will demonstrate the ability to replace the bevel gear, output shaft and select the shims to achieve proper backlash.

# Materials

Outlander V-series gearbox

Shift Lever

Output shaft shim set P/N 420281312

Bevel gear shim set (center case end) P/N 420281306

Bevel gear shim set (left case end) P/N 420281308

Caliper

0 – 1"inch Micrometer or a 0 – 25mm Micrometer

Screwdrivers

Feeler gauge set 0,05 to 1mm

Straight-edge

Backlash measurement tool – P/N 529036030

M10 X 16  Propellershaft Bolt

Magnetic base stand with dial gauge

Plastic mallet

Snap ring pliers

3/8 ratchet

3/8 extension

17mm nut

T-30 & T-40 Torx socket

Bevel gear and output shaft (replacement set)

Station Binder:

Shop Manual 219 100 542 Outlander Renegade 2012

Repair Order information

The customer complains there is a grinding noise in the gearbox area and the unit will not move.

Introduction

The disassembly, inspection and reassembly of the gearbox must be performed correctly.  It is important to carefully follow the instructions in the shop manual.  Pay attention to details when inspecting the shifter forks, shift drum, gears, bearings and shift shaft positioning or timing.  Make sure the transmission shifts at each step of the reassembly process.  Rotate the shafts and check for binding after each component is installed during reassembly.  When installing one case to another, hand-tighten the bolts and rotate everything before the torque is applied.  In this gearbox, there is a set of spiral bevel gears to transfer the power to the output shaft that applies the power to the drive shafts, differentials and finally to the wheels.  Accurate measurements must be taken in order to select the proper shims for the bevel gears.  The proper shims must be selected to achieve the proper backlash for quiet operation and long life.

NOTE: MY 06 Outlander 650 & 800 and MY 07 Outlander 800 & Renegade used the same high-gear ratio in the gearbox.  The gears had a ratio of 29 teeth to 34 teeth.

MY 07 Outlander 500 & 650 use a different high-gear ratio in the gearbox.  These gears have a ratio of 28 teeth to 35 teeth.  The result is a reduction in the RPM of the gearbox output shaft when HIGH is selected on these machines.

Procedures

|  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| - Using the shop manual, record the specification for output shaft backlash. NEW _____________ to ______________mm ( to in. ) SERVICE LIMIT _____________mm ( in. ) |  |  |  |  |  |  |
| - Record the specification for the distance mark on the backlash tool tab. _____________mm ( in. ) from the center of the bolt. |  |  |  |  |  |  |
| - Install the dial indicator on the measurement tool tab per the shop manual. See photo(s) this page and next. Shift the gearbox into PARK. Measure the backlash and record. Your reading _______________ |  |  |  |  |  |  |
| Note: The use of the backlash tool will pull the output shaft in the direction as the picture shown above when measuring the backlash. This is to take out any gaps between the output shaft assembly parts and to get the best measurement possible. |  |  |  |  |  |  |
| - Is the backlash within the specification? |  |  |  |  |  |  |
| - YES |  | NO |  |  |  |  |
| - Follow the procedures in the shop manual and remove the right housing of the gearbox. Pay attention to the orientation of the components as you disassemble them. Place the components on the bench in an orderly fashion that will make reassembly easier. |  |  |  |  |  |  |
| - Measure the coupling sleeve groove width. Is the coupler reusable? |  |  |  |  |  |  |
| - Measure the coupling fork claw thickness. Is the fork reusable? Note: For the purposes of this training, on this step do not remove the set screw and fork pin, just measure the fork in the housing. Also, do not reinstall the right side cover, in the following steps the transmission will continue to be disassembled. |  |  |  |  |  |  |
| - Measure the bevel gear axial clearance. Record your measurement? ______________ Note: This measurement is shown done with a feeler gauge in the shop manual section. It has been found to be easier and more accurate using the dial indicator for this measurement. |  |  |  |  |  |  |
| Follow the procedures in the shop manual and remove the left housing. Continue the disassembly of the gearbox. DO NOT REMOVE the index lever, parking lock lever, intermediate gear or any other component requiring a press operation. Notice the index marks on the shift shaft and shift drum. Pay attention to the orientation of the components as you disassemble them. Place the components on the bench in an orderly fashion that will make reassembly easier. Note: Shims are marked with a number, washers are not. The shop manual may refer to a shim as a thrust washer or vice versa. |  |  |  |  |  |  |

| - At this point you will replace the bevel gear shaft and output shaft with the other set at this work station. Verify that the bevel gear has an engraved number that matches a number engraved on the output shaft gear. These numbers indicate a matched gear set. |  |  |  |  |
| --- | --- | --- | --- | --- |
| - Perform the bevel gear adjustment procedure outlined in the shop manual to set the correct bevel gear depth and axial play for the replacement bevel gear and shaft. Use the chart on this page to record your measurements and calculate the thickness of the shims required. Refer to the image on page 219 of the 2012 shop manual to help you better understand the explanation of each measurement required. |  |  |  |  |
| - Your measurements are critical in setting the proper backlash between the bevel gears. You are likely familiar with the carpenter's adage, ‘’Measure Twice, Cut Once’’. Likewise, you will ‘’Measure Twice, Shim Once’’. Notice that there are two measurement columns in the chart, Measurement 1 and Measurement 2. Make each measurement twice and record. Verify the 2 measurements are correct and record the number in the CORRECT measurement column. Note: Keep all your measurements on the chart consistent so they are all in either standard or metric so the end values for the shims come out correct. |  |  |  |  |
| ## Letter | Measurement 1 | Measurement 2 | CORRECT Measurement |  |
| ## A | 2 mm (.0787) | 2 mm (.0787) | 2 mm (.0787) |  |
| ## B |  |  |  |  |
| ## C |  |  |  |  |
| ## D |  |  |  |  |
| ## E |  |  |  |  |
| ## F |  |  |  |  |
| ## G |  |  |  |  |
| ## H |  |  |  |  |
| ## I |  |  |  |  |
| ## J |  |  |  |  |

| - The "B" value is a given value. There is an engraved number on the bevel gear somewhere between -10 and +10. Use the formula from the shop manual procedure to determine the correct "B" value. "B" = ________ - Record this value in the measurement chart. |  |  |
| --- | --- | --- |
| - Use the photos below to determine where "C" is measured. The center housing should be oriented with the left housing mating surface facing "UP". |  |  |
| "C" = ________ - Record this value in the measurement chart. |  |  |
| 15. D = B – C – A "D" = ________ - Record this value in the measurement chart. |  |  |
| 16. Use your "D" measurement and the chart in the service manual to select shim "E". What is the thickness in inches or mm of the shim you selected for "E"? Note: Keep all your measurements on the chart consistent so they are all in either standard or metric so the end values for the shims come out correct. "E" = ________ - Record this value in the measurement chart. |  |  |
| 17. The "F" measurement is the distance between the mating surface of the left housing to the ball bearing inner race. Use the picture below to determine the "F" value. "F" = ________ - Record this value in the measurement chart. Note: Remember to subtract the bar thickness. |  |  |
| 18. The "G" measurement is the distance between the mating surface of the center housing and the shim butting surface. Use the picture below to determine the "G" value. "G" = ________ - Record this value in the measurement chart. Note: Remember to subtract the bar thickness. |  |  |
| 19. The "H" measurement is the distance between the thrust surfaces of the bevel gear shaft. Use the picture below to determine the "H" value. "H" = ________ - Record this value in the measurement chart. |  |  |
| 20. I = F + G – H – A – E "I" = ________ - Record this value in the measurement chart. |  |  |
| 21. Take the obtained theoretical thrust washer thickness "I" and choose the proper thrust washer number "J" according to the correct table in the shop manual. "J" = ________ - Record the correct shim to be used in the measurement chart. |  |  |
| 22. Follow the procedure in the service manual and reassemble the gearbox. After installing the output shaft be sure to seat the bearing in place per the instructions on page 209. |  |  |

| - Install the dial indicator on the measurement tool tab per the shop manual. Shift the gearbox into PARK position and measure the output shaft backlash and record. Your reading _______________ |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| - Is the backlash within the specification? |  |  |  |  |  |
| YES |  | NO |  |  |  |
| - If your backlash is NOT within specification, follow the procedure in the shop manual to obtain the backlash specification. Note: The shop manual does not mention the shims that are available to fine tune the output shaft. The shims that come in kit 420281312 are not listed separately. Here is a chart of each shim that comes in the kit. .070mm or .026"inches .080mm or .031"inches .090mm or .035"inches 1.00mm or .038"inches 1.10mm or .042"inches 1.20mm or .046"inches 1.30mm or .050"inches 1.40mm or .054"inches |  |  |  |  |  |
| - After you have obtained the proper specification, leave the dial indicator setup on the output shaft and answer the following questions. |  |  |  |  |  |

### Questions

1.  What have you learned from this task?

_______________________________________________________

_______________________________________________________

_______________________________________________________

2.  What sealant is used on the mating surfaces of the gearbox?

_______________________________________________________

3.  What sealant is used on the gearbox position and 4WD indicator switches?

______________________________________________________________________________________________________________

_______________________________________________________

4.  What type of lubricant is specified for the gearbox and coupling unit?

______________________________________________________________________________________________________________

5.  What is the difference in a MY 07 800 gearbox, versus, the MY 10 800 gearbox ?

______________________________________________________________________________________________________________

BE SURE TO STRAIGHTEN THE TOOLS AND THE WORKBENCH AREA

# Instructor sign off--  Go __________