| Unit |  | Task |  |
| --- | --- | --- | --- |
| 10 |  | 11 |  |
| Outlander MAX 800 LTD |  |  |  |
|  |  | ATV | Side-by-Side |

#### Charging System Testing

## Task Objective

At the completion of this task the technician will be able to properly check the output of the charging system.  In addition, he will be able to demonstrate the correct procedure to troubleshoot the three phase charging system using a multi-meter.  Further skills will be demonstrated by demonstrating how to check the amperage output.  He will also be able to demonstrate how to check for amperage draw.

# Materials

Task Sheet

# Shop manual

2 small vise grips

Amp probe

Multi-meter

Tachometer or B.U.D.S.

Stator connector P/N 529 036 016

Battery load tester

Crocodile clamps

10mm wrench

#### Repair order information

The customer states that the battery goes dead and needs to be recharged frequently.

#### Introduction

If a complaint of a battery going dead comes in, it is probably a charging system malfunction.  Or the person could be having an issue where a component is causing an excessive draw.  First, don’t forget to check the battery itself.  Then proceed with testing the charging system.

Find and disconnect the generator coil’s output leads from the vehicle’s wiring harness.  Set your multi-meter to the Ohms function and select the appropriate range (as per the resistance specification provided in the shop manual).  Connect the ohmmeter leads to the generator coil’s output wires.  Compare the ohmmeter reading to the specification.  If the reading is not within the specified range, replace the coil.

Next, you will want to make sure that the generator coil winding is not shorted to ground.  Perform this test by connecting one ohmmeter lead to one of the generator output wires and touching the other ohmmeter lead to ground.  If the generator is accessible, you can touch the ohmmeter lead to the generator rotor or backing plate.  If the generator is not accessible, touch the lead to any part of the engine or chassis that is grounded.

The results of this test should indicate infinite resistance.  In other words, there should be no continuity between the coil windings and ground.  If there is a measurable resistance, the winding is shorted to ground and must be replaced.

Even if the foregoing resistance test indicates normal resistance through the generator coil winding, that does not necessarily mean that the coil’s actual voltage output is sufficient.  In order to confirm that the coil actually is producing current, you will need to perform an AC output voltage test.

If the stator winding AC voltage output tests indicate that the stator assembly is functioning normally, the only remaining test to be performed is a DC output voltage output test of the rectifier/regulator.

To perform a rectifier/regulator DC output voltage test. Set your multi-meter to the DC volts function and select the appropriate range, as per the voltage output range specification provided in the vehicle’s shop manual (usually 0~20 VDC range).  Connect the positive (+) voltmeter lead to the regulator/rectifier output wire and the negative (-) voltmeter lead to the engine or chassis ground.  Start the engine and bring the engine’s RPM up to the test speed specified in the manual.  Compare the voltage reading to the specification.  If the reading is not within the specified range, replace the rectifier/regulator.

Loose or corroded circuit connections are likely the single most common cause of AC generator system malfunctions.  Always begin your troubleshooting with a thorough inspection of all circuit connections.  Don’t forget the ground connections.  Remember that ground connections are part of the circuit.

Also check the amp output.  An inductive ammeter is usually recommended for measuring current flow beyond 10 amps.  An inductive ammeter, for example, measures current flow through a circuit without actually having to be connected to the circuit.  As the name implies, an inductive ammeter measures current flow indirectly by sampling the magnetic field produced by current flowing through a circuit.

This is where the diagnosis begins.  You need to apply a logical method of diagnosis.  For example, first check the schematic and check the wire colors and locations of the terminals and components.  Visually check for disconnected connectors or backed out terminals.  Ensure there is enough grip on the terminal.  Ensure the wires are in the correct positions in the connector.  Ohm the component, first from the MPEM or ECM connector, then at the component.  Then verify the continuity of the wires from the component to the ECM.  Remember diagnosis is a process of eliminating what is good to determine what is bad.

## Charging System Testing

Refer to the proper Shop Manual for the correct procedure and setting specifications.

# Procedures

|  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| - Refer to the charging system section of the service manual and perform diagnostics. |  |  |  |  |  |
| - Refer to the tips provided. |  |  |  |  |  |
| - Specifications for load test? The battery should be tested at a ________ amp load load for a period of ________ seconds and the voltage should be not less than __________ volts. |  |  |  |  |  |
| - Load test the battery…..If you were in the field you would need to verify the condition of the battery before proceeding to any charging system test. For the purpose of this training, we will assume that the battery has passed a load test. |  |  |  |  |  |
| YES | X | NO |  |  |  |
| - Check the amperage output to the battery and record the amperage |  |  |  |  |  |
| - At what RPM and how many amperes would be considered a correct current test? |  |  |  |  |  |
| - Perform a voltage check at the battery as per the service manual and record the voltage? |  |  |  |  |  |
| - At what RPM is the voltage checked? |  |  |  |  |  |
| - What is the maximum voltage allowed to the battery? |  |  |  |  |  |
| - Test the stator as per the service manual and record the information below. |  |  |  |  |  |
| - What is the specification for static test for ohms? |  |  |  |  |  |
| - Stator leg #1 ohms? (Leg #1 = pin A to pin B) |  |  |  |  |  |
| - Stator leg #2 ohms? (Leg #2 = pin B to pin C) |  |  |  |  |  |
| - Stator leg #3 ohms? (Leg #3 = pin C to pin A) |  |  |  |  |  |
| - Stator to ground continuity? |  |  |  |  |  |
| - What is the specification for the dynamic test for voltage? |  |  |  |  |  |
| - Stator leg #1 voltage? (Leg #1 = pin A to pin B) |  |  |  |  |  |
| - Stator leg #2 voltage? (Leg #2 = pin B to pin C) |  |  |  |  |  |
| - Stator leg #3 voltage? (Leg #3 = pin C to pin A) |  |  |  |  |  |
| - What component have you determined to be faulty? |  |  |  |  |  |

### Questions

1.  What have you learned from this task?

___________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

2.  List possible causes of battery failure.

___________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

3.  How many amps does the vehicle draw, when not running and the key is off?

_______________________________________________________

4.  How many amps does the vehicle draw, when not running and the key is on?

_______________________________________________________

5.  How many amps does the vehicle draw, when not running and the key and          the lights are on?

_______________________________________________________

6.  How many AMPS does the vehicle charge when running?

_______________________________________________________

7.  What is the ampere per hour rating of the battery?

_______________________________________________________

Instructor sign off--  Go __________