| Station |  | Task |  |
| --- | --- | --- | --- |
| 12 |  | 1 |  |
| Stationary V-810 |  |  |  |
|  |  |  | ATV |

# 4-Stroke Cam Timing Procedures – V-810

## Task Objective

At the completion of this task the technician will be able to properly lock the engine crankshaft and camshaft.  He will also be able to perform the cam timing procedure in the correct sequence.

# Materials

# V810 4-TEC engine.

# Crankshaft locking tool: 529035617

# Camshaft locking tool: 5290035926

1/4", 1/2" drive ratchets

13mm and 15mm and 8mm wrench

Small flat blade screw driver

1/2" 14mm allen sockets, spark plug socket 16mm

½" extension and toggle

4 and 5 mm allen socket (1/4 drive)

T 30 socket

1/4"drive x 4" long extension

Feeler gauge

# Rags

Wire

Shop manual 219100273

# Introduction

Camshaft timing is obviously crucial to proper engine performance.  Proper camshaft timing ensures that the intake and exhaust valves are opened and closed at precisely the right time.  If the camshaft timing is incorrect, the valves may open and close too soon or too late.  This will result in poor engine performance, (if the engine is able to start at all).  If valve timing is seriously out of synchronization with the rotation of the crankshaft, the valves may open when the piston is approaching TDC, and the piston will collide with the valves.  This will result in bent valves, which will no longer seat or seal properly.  Serious piston damage can also result.  In either case, once the valves are bent, the engine will no longer run.

Careful and accurate cam timing is particularly important in dual over-head cam (DOHC) engines.  Since DOHC engines use separate intake and exhaust cams, it is crucial that the position of both cams be carefully synchronized with the crankshaft.  Otherwise, not only can the valves collide with the pistons but the intake and exhaust valves could collide with one another.

# Camshaft Timing

# Refer to the appropriate shop manual as listed in the Materials section.  One of the first steps in performing any procedure is to locate the information and read it prior to performing the procedures.  The technician should ensure all necessary tools are available.  The shop manual is the best resource for information, specifications and step by step instructions pertaining to all aspects of the vehicle.  In summary, the importance of the shop manual can not be overstated and its use will always be the required first step of any procedure.

# Procedures

|  |  |
| --- | --- |
| - Follow the procedures provided in this task sheet and the shop manual to complete the repair and record the following information. |  |
| - Lock cylinder number 2 at TDC. Make sure the locking bolt engages the groove in the crankshaft. Note: Cylinder number 1 cannot be locked at TDC using the crankshaft locking tool. |  |
| - Remove the chain tensioner for cylinder number 2. |  |
| - Use the camshaft locking tool, lock the camshaft at TDC. |  |
| - Why must the camshaft locking tool be used? |  |
| - Remove the camshaft timing gear. Make sure to use a piece of wire to secure the timing chain from dropping into the engine bottom half. |  |
| - Now rotate the crankshaft counterclockwise (approximately 280°) until the marks on the magneto flywheel number 1 and the magneto cover, are aligned. Extreme care needs to be taken so that the timing chain for cylinder number 2 does not get hung up or stretched. |  |
| - Remove the chain tensioner for cylinder number 1. |  |
| - Use the camshaft locking tool and lock the camshaft at TDC. Once locked, remove the gear as you did in step # 6. Remember to use a wire to secure the timing chain. |  |
| - Remove the camshaft retainers, then the rocker shafts and the rocker arms from each cylinder. |  |
| - Remove both camshafts. Caution: The camshafts are not identical in design. Do not mix-up the camshafts. Any mix-up of the components will lead to engine damage. |  |
| - Carefully rotate the crankshaft clockwise and lock cylinder number 2 at TDC using the crankshaft locking bolt. Both timing chains need to be monitored closely so they do not jump off their respective crank gear, hang up or drop into the bottom end. |  |

STOP! 	Instructor Check Point	____________

| 13. Re-install the camshaft for cylinder number 2. |  |
| --- | --- |
| 14. Re-install the rocker arms and shafts for cylinder number 2. |  |
| 15. Install the camshaft retainer. |  |
| 16. What is the torque for the camshaft retainer bolt? |  |
| 17. Re-install the camshaft timing gear and chain as per the shop manual for cylinder number 2. |  |
| 18. What is the torque for the camshaft timing gear bolt? |  |
| 19. Which cylinder has the trigger wheel? |  |

| - Install the timing chain tensioner for cylinder number 2 paying close attention to the procedure in the shop manual. Once there is no longer any back and forth movement of the timing gear, the plunger should be turned_______to reach the required torque of_________. Failure to accomplish this procedure correctly will lead to___________________? |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 21. Cylinder #2 timing marks should be parallel with the top of the cylinder head. |  |  |  |  |  |
| 22. Is the installation correct? |  |  |  |  |  |
| YES |  | NO |  |  |  |
| 23. Remove the crankshaft locking bolt and turn the engine counterclockwise as viewed from the magneto side of the engine (approximately 280°) until cylinder number 1 is at TDC. |  |  |  |  |  |
| 24. Re-install the camshaft for cylinder number 1. |  |  |  |  |  |
| 25. Re-install the rocker arms and shafts for cylinder number 1. |  |  |  |  |  |
| 26. Install the camshaft retainer. |  |  |  |  |  |
| 27. Re-install the camshaft timing gear and chain as per the shop manual for cylinder number 1. |  |  |  |  |  |
| 28. Install the timing chain tensioner for cylinder number 1 paying close attention to the procedure in the shop manual. Once there is no longer any back and forth movement of the timing gear, the plunger should be turned_______to reach the required torque of_________. Failure to accomplish this procedure correctly will lead to___________________? |  |  |  |  |  |

| 29. Cylinder number 1 timing marks should be parallel with the top of the cylinder head. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 30. Is the installation correct? |  |  |  |  |  |
| YES |  | NO |  |  |  |
| 31. With cylinder number 1 at TDC, cylinder number 2 should appear to have the timing marks in parallel to the table. They are inversely proportional. |  |  |  |  |  |
| 32. Remove the camshaft locking tool and make sure the engine rotates freely. |  |  |  |  |  |
| 34. Perform the valve adjustment as per shop manual. |  |  |  |  |  |
| 33. Clean up the workstation. |  |  |  |  |  |

### Questions

1.  What have you learned from this task?

______________________________________________________________________________________________________________

_______________________________________________________

2. What would happen if this procedure where not done correctly?

_____________________________________________________________________________________________________________________________________________________________________

- Is there any visible difference between front and rear cylinder cam? If yes, can you describe the most obvious?

____________________________________________________________________________________________________________________________________________

Please type in the correct cylinder number:

Instructor sign off--  Go __________