| Unit |  | Task |  |
| --- | --- | --- | --- |
| 8 |  | 1 |  |
| Running 800 HO Engine |  |  |  |
|  |  | ATV | Side-by-Side |

BUDS Navigation

Task Objective

At the completion of this task the technician will be able to properly utilize BUDS to navigate through the various screens and functions available.  In addition he will be able to explain when this procedure would be necessary.  He will also be able to explain the importance of the having the correct wiring to the correct sensor and how that can be tested.

Materials

BUDS

MPI 2

Multimeter

Small screw driver

ECM adaptor tool

Wiring diagram 2011

Station binder and Shop Manual 2007 or up

#### Repair Order Information

The customer would like the technician to just check his system with the BUDS diagnostic computer just to see if everything is in good condition.

Introduction

The scope of this document is to present the different service functionality of the Outlander 800

Note: This document is a complement to the service manual for the Outlander 800.

Electrical system

Cam Sensor (CAPS)

Located on the side of the front cylinder (cylinder #1), it is used to synchronize the engine crankshaft with the camshaft by noting the position of the camshaft.

Speed Sensor (CV)

Located on the right side of the sub-transmission behind the actuator for 4WD, it reads the passage of 4 embosses on the transmission shaft using a magnetic field.

2WD/4WD Actuator Switch (SD)

Located on the sub-transmission right side the switch is reading the position of the 4WD actuator.

Note1: When in 2WD, the switch gives a GND and is open in 4WD.

Crankshaft Position Sensor (CPS)

Located on the right side of the engine on the magneto cover, it gives the crank position and speed to the ECM.

Idle Bypass Valve (IDLE)

Located on the throttle body, it acts as a secondary input of air to influence the engine behavior.  It is a stepper motor controlling a plunger.  There is no test at this time available in BUDS to test the functionality of the sensor.  The only way to validate that the IDLE is operational is to listen to it and feel it when powering up the vehicle.

Manifold Air Pressure and Temperature Sensor (MAPTS)

Located on the Intake manifold, it reads the air temperature and pressure going to the engine. Note: The pressure and temperature are not the same as the ambient because of the throttle body and manifold effect and temperature emitted by the engine.

Throttle Position Sensor (TPS)

Located on the throttle body, it reads the exact position of the throttle body.

Gear Position Switches (PRN)

Located on the sub-transmission right side the switches (3) are reading the position of the gear (P, R, N, H and L).  Note, the position is read in combination where not one is specific for one gear position only.

Coolant Temperature Sensor (CTS)

Located on the left side of the engine on the rear cylinder (cylinder #2), it is reading the engine coolant temperature for the engine.

Oil Pressure Switch (OPS)

Located on the right side of the engine in over the magneto cover, under a plastic cover, it is reading the engine oil pressure.  When the oil level or pressure is low the switch is "closed", and is "open" when the oil pressure is OK.

Communication

The vehicle is equipped with a communication network called CAN (Controller Area Network) and the equipment it communicates with are the ECM, the multi-function speedometer and VCK.  All information relative to the diagnostics, programming and history are exchanged.

Between the ECM and Speedometer the following information is exchanged:

• Vehicle speed

• Engine RPM

• Low battery voltage

• High battery voltage

• Key status

• Check engine

• Maintenance soon

• Low oil

• High temp

• Limp home

• Engine hour meter

• Sub-transmission position

• P-codes

• Security exchange

Regulator/Battery and set voltage

The Outlander 800 can not run without a battery. It is necessary to have a voltage to the ECM that is 7V DC minimum to work.  The regulator output is set to 14.84V and varies slightly depending on the loads, engine RPM, battery state and temperature.

DESS key

The ignition key is equipped with the DESS technology.  Mechanically, all the keys have the same teeth pattern but are different to each other by the digitally encoded chip integrated in each one.

If the DESS key is left for more then 15 minutes in the "On" position the ECM will enter a Sleep mode where only the multi-function speedometer, tail lamp and ECM will be powered but the engine cannot be started anymore.  Return to a normal operation can only be done by turning the key to the "Off" position, wait for the Speedometer to shut down and powering it back.

Fuse box

The main fuse box is located in front of the vehicle in the service compartment.  The second is located on the right side of the battery holder.

The fuse box in the rear contains two or three fuses (depends on the model), one main for accessories (F9) and the other for the main function (F8) of the vehicle, both fuses are 30 Amp. The 3rd fuse on DPS equipped model is the DPS main fuse, 40 Amps.

The main fuse box located in the front of the vehicle contains an assortment of fuses, diodes and relays plus a removal tool.

High lights of the front fuse-block:

The diode D1 is used to prevent voltage return from the ECU from pin 26 to pin 17 that would maintain the main relay powered at all time.  If the Diode is inverted (installed backwards) or not connected, the vehicle will not power-up.

The diode D2 (on current models installed in the wiring harness, NOT in fuse box anymore!) is called a free wheel diode because the Starter Solenoid will generate an inverted voltage peak that could damage other components.  This diode will simply short this inverted reverse pulse and prevent damage to other components.

Note in the picture, the orientation of the direction of the diodes and also the relays.

Fuse Box PF1

- R1: Cooling Fan

- R2: Main Relay

- R3: Accessories Relay

- D1: ECM Diode

- F1: Ignition Coils – 5 amp

- F2: Cooling Fan – 20 amp

- F3: Injectors – 5 amp

- F4: Cluster – 7.5 amp

- F5: Fuel Pump – 7.5 amp

- F6: ECM – 5 amp

- F7: Accessories – 20 amp

- F10: Horn/ Flasher(EC) – 5 amp

ECM

The ECM is separated in three main functionalities:

Vehicle function

The ECM controls some of the vehicle function that the user only interfaces.

Power up

The vehicle is powered by a combination of the ignition key (DESS) to any of the "On" position and the Run switch to the "Run" position.  This will give permission to the ECM to maintain the main relay.  Once one of the first conditions is not valid, the ECM will record the last data for a certain time and release the main relay (approximately 15 seconds).

DESS reading

At the moment of power up, the ECM will read the digitally encoded chip in the key and validate that the corresponding number has been previously recorded in memory.  If it is not present, a fault will be recorded, the check engine lamp will be powered, a message will be displayed and all starting functions will be deactivated.  If the key is not read, a fault will be recorded, the check engine lamp will be powered, a message will be displayed and all starting functions de-activated.  In the case that the key is in memory, the ECM will give authorization for all starting sequences.

Security exchange

The security exchange is a function that prevents the end user to mix this ECM with one from another engine because they all look the same.  The security exchange code is a code recorded by the factory or by BUDS (for replacement or repairs) into the multi-function speedometer.  This code is sent periodically by the Speedometer to the ECM.  The ECM only needs to read the code once.

If the code does not match the required one in the ECM, it simply prevents any starting process.  If the code is absent, example the gauge is broken or removed; the ECM will authorize the starting process.  It will also be the same with the proper code.

Starting sequence

The ECM controls the Starter Solenoid output and will power it up only if the following set functions are met:

• Ignition key in the running position

• DESS is ok

• Security exchange is ok

• Sub-trans is in Park or Neutral or if the Sub-trans in any position and the brake is activated

While starting and under 800 RPM approximately, the ECM will shut down the accessory relay to give more energy for starting available.

Engine cooling fan

The ECM will activate the cooling fan when the temperature of the engine coolant has reached a temperature specification of around 100ºC or 212ºF.

Pilot Lamp and messages

The ECM will send information to display the "Check Engine" lamp when a fault occurs or that a limp home mode has been activated.  It will also trigger most of the different messages displayed in the Multi-function Speedometer.  Finally, it will broadcast periodically all the active P codes on the CAN bus for the speedometer to retrieve.

Sleep mode

When the ignition key and the engine stop switch are left in the running position with the engine not running for more then 15 minutes, the ECM will engage a sleep mode sequence in order to prevent starting and limiting power consumption.  At this point only the Speedometer, tail lamp and ECM will remain powered. The starting process will be disabled.  To restore all functions, the vehicle must be completely shut down and turned back "On" again.

Drowned mode

In case the engine has been flooded with fuel, it is possible to activate the starter without injecting fuel.  If the throttle body has been engaged over 2/3 of it travel, the ECM will go into a function called Drowned mode.  It will permit the cranking part of the starting process to occur, but will prevent fuel injection and ignition.

Engine revolution limiter

The engine revolution limiters are as follow:

• Low battery voltage @ 5000 RPM - (voltage below 11V)

• High battery voltage @ 7000 RPM - (voltage over 15.992V)

• High engine temperature @ 4050 RPM - (temperature over 118 degrees C or 244 degrees F)

• Safety fuel cut off (TPS error or internal memory error @ 5000 RPM

• Low Oil @ 4000 RPM

Fuel pump

The fuel pump is controlled directly by the ECM.  When the vehicle is powered (ignition key and engine stop switch in running position) the pump is activated for a certain period to pressurize the injectors to be ready to operate.  Once this sequence has been completed, the fuel pump will be activated every time the start button is pressed even if the starter is not engaged for any of the previously presented reason (see starting process).  It will continuously operate once the engine is starting to turn (including starting).

Diagnostic

The last but important function of the ECM is the diagnostic of the engine and of most of its inputs and outputs.

Engine

The ECM will read the signals from its inputs to monitor the engine to see that it operates within set parameters.  If the information detects a failure while the engine is running, the ECM will record the fault (in most cases), time of failure and the amount of occurrence.  It will also depending on the failure, take action in reducing performance of the engine.  This would be done by limiting its maximum RPM or this is also called going to "limp home mode".  The unit informs the user by powering up the "Check Engine" pilot lamp and in some cases making it blink when in limp home mode.

Example 1 - Low pressure of the engine oil will activate an engine revolution limiter, the check engine pilot lamp to flicker and a text message to indicate "Low Oil".

Example 2 - One of the injectors is not operational.  The "Check Engine" pilot light will power up but will not blink, no message will appear and no revolution limiter will be in action.  At this point the engine is not running properly by the result of fuel not present in that cylinder.

Input sensors and outputs

The ECM will continuously check each input/output to validate that they operate within the set parameter.

Multi-function Speedometer

This Multifunction Speedometer comprises three main features: One feature is the showing of Speed or RPM. The second feature is the majority of the information will be coming from a CAN line (communication network between the Multifunction Speedometer and the Ignition module).  The third feature is the engine fault codes are available by a back door program.

The speedometer button

The mode switch, located in the Multi-function Speedometer, will toggle through the functions in Section 1 of the LCD (Odometer, Trip meter, Hour meter, ECM hour meter and RPM).

In "Trip" meter, by a 2 seconds activation of the switch, we will reset the trip meter to zero.

In "Hour" meter, by a 2 seconds activation of the switch, will reset the hour meter to zero.

Engine fault back door program (P codes displayed on speedometer)

With engine hours displayed, by putting the key switch in light “ON” position, a 2 seconds activation of the switch and simultaneously toggle the head lamp between High and Low three times, the LCD 2 will display the P codes.  Activate the switch to toggle trough active P codes.  If END is displayed, the last P code was displayed.  To switch back to normal mode, maintain the button over 2 seconds.

# Procedures

Refer to the proper Station Binder section and the schematic and use a logical method for diagnosing this circuit malfunction.

####   Place the switches on the switch box in the following positions.

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | X | X | X | X | X | X | X | X |
| B |  |  |  |  |  |  |  |  |

|  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - Connect BUDS to the vehicle. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Install the DESS key on the vehicle and turn the key to the run position. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Ensure that the shut down switch is in run position. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Read the BUDS data. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Go to the "vehicle" screen. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Record the engine number. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Record the date and hours of the last service. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Go to "keys" screen. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Record the number of keys used. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Go to the "settings" screen. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Record the degrees of throttle opening. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Change the cluster scale from either KPH to MPH or MPH to KPH. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Go to "monitoring" screen page "Cluster" |  |  |  |  |  |  |  |  |  |  |  |  |
| - Push the cluster mode button on the vehicle and observe the indicator light of the BUDS screen. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Does the light illuminate? |  |  |  |  |  |  |  |  |  |  |  |  |
| YES |  |  | NO |  |  |  |  |  |  |  |  |  |
| - Switch the 2x4 switch to the 4x4 position and observe the indicator light at the bottom of the BUDS screen. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Does the light illuminate? |  |  |  |  |  |  |  |  |  |  |  |  |
| YES |  |  | NO |  |  |  |  |  |  |  |  |  |
| - Go to the "monitoring" screen page ECM |  |  |  |  |  |  |  |  |  |  |  |  |
| - Place the cursur over the gauges on the BUDS screen and record the following. --- |  |  |  |  |  |  |  |  |  |  |  |  |
| - Intake air temp |  |  |  |  | Value |  |  |  |  | MIN | Max |  |
| - Engine temp |  |  |  |  | Value |  |  |  |  | MIN | Max |  |
| - MAPS |  |  |  |  | Value |  |  |  |  | MIN | Max |  |
| - Volts engine off. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Start the engine. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Volts running |  |  |  |  |  |  |  |  |  |  |  |  |
| - Shut down the engine using the BUDS red "engine stop" icon at top of screen. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Place the vehicle into drowned mode and crank. |  |  |  |  |  |  |  |  |  |  |  |  |
| - What BUDS indicator lights illuminate while cranking? |  |  |  |  |  |  |  |  |  |  |  |  |
| - Start the engine and perform "shutdown cylinder" and record the following. |  |  |  |  |  |  |  |  |  |  |  |  |
| - RPM drop, front cylinder fuel only? |  |  |  |  |  |  |  |  |  |  |  |  |
| - RPM drop, rear cylinder fuel only? |  |  |  |  |  |  |  |  |  |  |  |  |
| - RPM drop, front cylinder fuel and ignition? |  |  |  |  |  |  |  |  |  |  |  |  |
| - RPM drop, rear cylinder fuel and ignition? |  |  |  |  |  |  |  |  |  |  |  |  |
| - Shut down the engine using the BUDS red "engine stop" icon at top of screen. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Go to the "activation" screen page ECM |  |  |  |  |  |  |  |  |  |  |  |  |
| - Take care for a quite environment and listen to the front injector |  |  |  |  |  |  |  |  |  |  |  |  |
| - Activate the front cylinder ignition coil. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Does front cylinder ignition coil activate? |  |  |  |  |  |  |  |  |  |  |  |  |
| YES |  |  | NO |  |  |  |  |  |  |  |  |  |
| - Take care for a quite environment and listen to the rear spark plug |  |  |  |  |  |  |  |  |  |  |  |  |
| - Activate the rear cylinder ignition coil. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Does it activate? |  |  |  |  |  |  |  |  |  |  |  |  |
| YES |  |  |  | NO |  |  |  |  |  |  |  |  |

####   Place the switches on the switch box in the following positions.

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A |  | X | X | X | X | X | X | X |
| B | X |  |  |  |  |  |  |  |

| - Go to the "faults" screen. |  |
| --- | --- |
| - Record the faults and the descriptions here. |  |
| - Is the fault active or occurred? |  |
| - How many times did the fault occur? (counts) |  |
| - At how many minutes did the fault occur? (total time) |  |
| - What is the total hours on the machine? (from the vehicle screen) |  |
| - Go to "more details" of the first fault. |  |
| - Record the service actions here. |  |
| - What are the wire colors and locations at the ECM for that circuit? (refer to the schematic) |  |
| - Go to the "monitoring" screen and view the faulty sensors data and record. |  |
| - What needs to be repaired on the unit? Be specific list circuit numbers, color codes and any other pertinent information. |  |

####   Place the switches on the switch box in the following positions.

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | X | X | X | X | X | X | X | X |
| B |  |  |  |  |  |  |  |  |

| - Clear the occurred faults |  |
| --- | --- |
| - Switch on again the ignition |  |
| - Re-read the faults. |  |
| - Did faults return? |  |
| - Go to the "history" screen. |  |
| - Go to the "RPM profile". |  |
| - Enter the total history percentage of time the vehicle was operated between 3100 and 4500 RPM 3001-4500 = _______% |  |
| - Go to "last minute". |  |
| - On bottom left of BUDS screen change view to "speed and throttle opening". |  |
| - Record peak speed (RPM) and peak TPS (degrees) - RPM =____________ Degrees = ____________º |  |
| - NOTE: The last minute data must be cleared before a re-read of the data can occur. |  |
| - On bottom left of BUDS screen change view to "last minute history table." |  |
| - Go to "print". |  |
| - Enter your name and any other comments you desire into the comments section of the service report. |  |
| - Disconnect BUDS |  |

# Questions

1.  What have you learned from this task?

_____________________________________________________________________________________________________________________________________________________________________

### Instructor sign off--  Go __________

| Unit |  | Task |  |
| --- | --- | --- | --- |
| 8 |  | 2 |  |
| Running 800 HO Engine |  |  |  |
|  |  | ATV | Side-by-Side |

##### Output Fault Diagnosis

# Fuel injector resistance test

## Task Objective

At the completion of this task the technician will be able to properly utilize BUDS to activate the output devices on the Outlander 800 HO engine.  In addition he will be able to explain when this procedure would be necessary.  Further skills will be demonstrated by the correct diagnosis of a failed output driver circuit.  He will also be able to explain the importance of the having the correct wiring to the correct component and how that can be tested.  In addition, the technician will be able to properly perform an injector resistance test.  Further skills will be demonstrated by utilizing BUDS testing procedures to find the correct specifications and testing points.  He will also be able to explain why an injector resistance test would be necessary.

# Materials

Multi-meter

BUDS

MPI 2

ECM Diagnostic Box

Task sheet

Station binder and Outlander 800 Shop Manual 2007 or up

#### Repair Order Information

The customer states the engine is not running correctly.  He says it feels like it is running on less than all the cylinders.  Further diagnosis has shown good compression tests.  The engine has spark.

###### Introduction

Understanding the B.U.D.S system and its options is important for the proper entry of such information as customer name and delivery date.  The B.U.D.S system is also used for the programming of DESS (Digital Encoded Security System) lanyards to the vehicle.  It can also be used for setting the ignition timing and TPS (Throttle Position Sensors) on some models and reading fuel injection diagnostic information on equipped models.

In overview, the B.U.D.S. system will allow you to read the information stored in the vehicle's MPEM and/ ECM, modify the parameters in the B.U.D.S program, save the entire B.U.D.S file to disk or hard drive, print a service report and write the information back into the vehicle's MPEM.

When working with BUDS on a fuel injected engine are several diagnostic functions that can be utilized.  Access to fault codes and diagnostic information can be found in the faults screen.  In addition there are several items that can be activated by the system.  Not only can items be activated but, in some case items can be deactivated.

When the engine is not running, certain items can be activated.  Some items that can be activated include the fuel pump, fuel injectors and ignition coils.  This function is useful to determine if the components are operating.  By “feeling” the activated components you can also determine if the correct component is being activated.  This eliminates crossed connectors to various output components.

When the engine is running certain items can be deactivated.  Items that can be deactivated include the ignition coils and injectors.  This function is on the monitoring page under cylinder shut down.  This is very useful for performing a cylinder contribution test.

The injector itself is an electrically operated solenoid.  Inside the injector is a set of windings around a tube that is filled with pressurized fuel.  Also inside the tube is a pintle that is held down by a return spring to close off the outlet hole of the injector.  When an electrical current is applied to the injector, current flows through the set of windings.  There is a magnetic field created that acts on the pintle and pulls the tip out of the outlet hole.  Fuel then sprays from the nozzle.  When the current is turned off the magnetic field collapses and the pintle is pulled back down to block off the outlet hole by the return spring.

# Fuel Injector Resistance Test

Refer to the proper Station Binder section, BUDS and this task sheet for the correct testing procedure and specifications.

# Procedures

####   Place the switches on the switch box in the following positions.

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | X |  | X | X | X | X | X | X |
| B |  | X |  |  |  |  |  |  |

|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - Connect BUDS to the vehicle by plugging into the 6 pin adapter. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Install the DESS key in on vehicle. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Turn on Ignition switch. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Read the BUDS data. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Save Buds data to your desk top. Ad your name at the end of the file name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Check the faults. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Record the faults and the descriptions here. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Go to the "activation" page. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Place the cursur over each red component and activate them. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Did all the components activate? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - YES |  |  |  | NO |  |  |  |  |  |  |  |  |  |  |  |
| - If not, which one did not activate? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - What are the color codes of the wires going to the components that did not activate? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - What are the terminal locations of the wires at the ECM location? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - What fuse protects this circuit? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Which component supplies the power? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Which component supplies the ground? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Check the specifications and test procedures at the "faults" screen. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - What is the spec in ohms of the component? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Install the ECM Diagnostic Box. Keep in mind to always shut down the unit first!!!!!! |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - With the multi-meter check the reading from the ECM connector and the fuse terminal. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Record the result here. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Is the reading correct? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - YES |  |  |  |  | NO |  |  |  |  |  |  |  |  |  |  |
| - With the multi-meter check at the component. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Is the reading correct? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - YES |  |  |  |  | NO |  |  |  |  |  |  |  |  |  |  |
| - Check the continuity of the wires from the component to the fuse terminal. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Is the continuity correct? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - YES |  |  |  |  |  | NO |  |  |  |  |  |  |  |  |  |
| - Check the continuity of the wires from the component to the ECU. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - What component have you determined to be faulty? Be specific list circuit numbers, color codes and any other pertinent information. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Remove the diagnostic box and reconnect all connectors. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### Place the switches on the switch box in the following positions.

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | X | X | X | X | X | X | X | X |
| B |  |  |  |  |  |  |  |  |

| - Using BUDS clear all active and occurred faults. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| - Did all faults clear? |  |  |  |  |  |
| - YES |  | NO |  |  |  |
| - Disconnect BUDS. |  |  |  |  |  |

### Questions

1.  What have you learned from this task?

______________________________________________________________________________________________________________

2.  Why would the component activation function be utilized?

3.  BUDS will tell you exactly what is wrong with the components and the vehicle.

TRUE

FALSE

Instructor sign off--  Go __________

| Unit |  | Task |  |
| --- | --- | --- | --- |
| 8 |  | 3 |  |
| Running 800 HO Engine |  |  |  |
|  |  | ATV | Side-by-Side |

# Input Fault Diagnosis

## Task Objective

At the completion of this task the technician will be able to properly test an input sensor on a fuel-injected unit.  During the procedure he will navigate through the BUDS system to obtain codes.  In addition, he will be able to explain the how to interpret those codes.  He will also demonstrate his ability to accurately follow the diagnostic procedure.  In addition he will be able to demonstrate the proper logical trouble shooting procedures.  Further skills will be demonstrated by properly clearing the codes when the repair is complete.

# Materials

Task Sheet

BUDS

MPI 2

Multi-meter

Station binder and Shop Manual 2007 or up

#### Repair Order Information

#### Unit produced a fault code, check engine light appears

###### Introduction

The BRP's Utility and Diagnostic Software, (B.U.D.S.), gives you an inside view of BRP's, ECUs.

Access to fault codes and diagnostic information can be found in the faults screen.  In addition, there are several items that can be activated by the system.  Not only can items be activated but, in some case items can be de-activated.

The newer MPEM generation on injection models can register faults.  When a vehicle has a problem, it is very helpful to consult these.  They can also be used to confirm that the repair was successful, by clearing occurred faults, and checking if they come back.

In addition by double clicking on the code a list of possible causes and service actions can be accessed.  This aids in trouble shooting.  However, it is only a pointer in the direction of the fault.

It may simply say check the circuit number X_XX.  Also it may possibly give you resistance or voltage readings.

Now the diagnosis begins.  You need to apply a logical method of diagnosis.  For example, first check the schematic and check the wire colors and locations of the terminals and components.  Visually check for disconnected connectors or backed out terminals.  Ensure there is enough grip on the terminals.  Ensure the wires are in the correct positions in the connectors.  Test any power source and, or ground to the component at the component.  If any resistance values exist for the component, check those values at the component and if in specification, continue testing in the direction of the ECM connectors or ECM.  Remember, disconnecting or replacing the ECM should only be done as a last resort.  When it is necessary to disconnect the ECM, ensure to verify the continuity of the wires from the component to the ECM.  In conclusion, diagnosis is simply a process of eliminating what is good to determine what is bad.

Note: The ECM connector terminals are very delicate and therefore the ECM connections should never be removed unless absolutely necessary.  Perform as many tests as possible without disrupting the ECM connectors.

## Input Fault Code Diagnosis

Refer to the proper Station Binder section and the schematic for the correct procedure and setting specifications.

# Procedures

#### Place the switches on the switch box in the following positions.

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | X | X |  | X | X | X | X | X |
| B |  |  | X |  |  |  |  |  |

|  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| - Connect BUDS to the vehicle. |  |  |  |  |  |
| - Install the DESS key on the vehicle. |  |  |  |  |  |
| - Read the BUDS data. |  |  |  |  |  |
| - View the "monitoring" screen. |  |  |  |  |  |
| - Is there any sensor data that looks out of range? |  |  |  |  |  |
| - YES |  | NO |  |  |  |
| - If so, list the incorrect data. |  |  |  |  |  |
| - Record the faults and the descriptions here. |  |  |  |  |  |
| - What needs to be repaired on the unit? Be specific list circuit numbers, color codes and any other pertinent information. |  |  |  |  |  |

####   Place the switches on the switch box in the following positions.

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | X | X | X | X | X | X | X | X |
| B |  |  |  |  |  |  |  |  |

| - Using BUDS clear all active and occurred faults. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| - Did all the faults clear? |  |  |  |  |  |
| - YES |  | NO |  |  |  |
| - Disconnect BUDS. |  |  |  |  |  |

### Questions

1.  What have you learned from this task?

_____________________________________________________________________________________________________________________________________________________________________

2.  List possible causes for failure.

_____________________________________________________________________________________________________________________________________________________________________

Instructor sign off--  Go __________

| Unit |  | Task |  |
| --- | --- | --- | --- |
| 8 |  | 4 |  |
| Running 800 HO Engine |  |  |  |
|  |  | ATV | Side-by-Side |

# Fault Code Diagnosis

## Task Objective

At the completion of this task the technician will be able to properly perform various electrical circuit diagnostics by methods of dynamic testing and using input verses output strategies.  In addition, the student will be able to demonstrate how to perform a logical approach on an electrical diagnostic situation.

Materials

BUDS

MPI 2

Schematic

ECM diagnostic tool

Station binder and matching shop manual.

#### Repair Order Information

#### The customer states that there is an "E" in gear indicator on cluster

Introduction

Now the diagnosis begins.  You need to apply a logical method of diagnosis.  For example, first check the schematic and check the wire colors and locations of the terminals and components.  Visually check for disconnected connectors or backed out terminals.  Ensure there is enough grip on the terminals.  Ensure the wires are in the correct positions in the connectors.  Test any power source and, or ground to the component at the component.  If any resistance values exist for the component, check those values at the component and if in specification, continue testing in the direction of the ECM connectors or ECM.  Remember, disconnecting or replacing the ECM should only be done as a last resort.  When it is necessary to disconnect the ECM, ensure to verify the continuity of the wires from the component to the ECM.  In conclusion, diagnosis is simply a process of eliminating what is good to determine what is bad.

Note: The ECM connector terminals are very delicate and therefore the ECM connections should never be removed unless absolutely necessary.  Perform as many tests as possible without disrupting the ECM connectors.

# Testing

Refer to the proper Station Binder section and the schematic and use a logical method for diagnosing this circuit malfunction.

# Procedures

####   Place the switches on the switch box in the following positions.

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | X | X | X |  | X | X | X | X |
| B |  |  |  | X |  |  |  |  |

|  |  |
| --- | --- |
| 1. Connect BUDS to the vehicle |  |
| 2. Install the DESS key in the vehicle. |  |
| 3. Read the BUDS data. |  |
| 4. Check the faults. |  |
| Record the faults and the descriptions here if there are any. |  |
| 5. Perform a logical method of diagnosis for this failure. |  |
| What needs to be repaired on the unit? Be specific list circuit numbers, color codes and any other pertinent information. |  |

####   Place the switches on the switch box in the following positions.

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | X | X | X | X | X | X | X | X |
| B |  |  |  |  |  |  |  |  |

| 6. Clean up the work station, clear all fault codes |  |
| --- | --- |

### Questions

1.  What have you learned from this task?

___________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Instructor sign off--  Go __________

| Unit |  | Task |  |
| --- | --- | --- | --- |
| 8 |  | 5 |  |
| Running 800 HO Engine |  |  |  |
|  |  | ATV | Side-by-Side |

# Fault Code Diagnosis

## Task Objective

At the completion of this task the technician will be able to properly perform various electrical circuit diagnostics by methods of dynamic testing and using input verses output strategies.  In addition, the student will be able to demonstrate how to perform a logical approach on an electrical diagnostic situation.

Materials

BUDS

MPI 2

Schematic

ECM diagnostic tool

Station binder and matching shop manual.

#### Repair Order Information

#### The customer complained that unit is not working anymore

Introduction

Now the diagnosis begins.  You need to apply a logical method of diagnosis.  For example, first check the schematic and check the wire colors and locations of the terminals and components.  Visually check for disconnected connectors or backed out terminals.  Ensure there is enough grip on the terminals.  Ensure the wires are in the correct positions in the connectors.  Test any power source and, or ground to the component at the component.  If any resistance values exist for the component, check those values at the component and if in specification, continue testing in the direction of the ECM connectors or ECM.  Remember, disconnecting or replacing the ECM should only be done as a last resort.  When it is necessary to disconnect the ECM, ensure to verify the continuity of the wires from the component to the ECM.  In conclusion, diagnosis is simply a process of eliminating what is good to determine what is bad.

Note: The ECM connector terminals are very delicate and therefore the ECM connections should never be removed unless absolutely necessary.  Perform as many tests as possible without disrupting the ECM connectors.

# Testing

Refer to the proper Station Binder section and the schematic and use a logical method for diagnosing this circuit malfunction.

# Procedures

####   Place the switches on the switch box in the following positions.

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | X | X | X | X |  | X | X | X |
| B |  |  |  |  | X |  |  |  |

|  |  |
| --- | --- |
| - Perform a logical method of diagnosis for this failure. |  |
| - What needs to be repaired on the unit? Be specific list circuit numbers, color codes and any other pertinent information. |  |

####   Place the switches on the switch box in the following positions.

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | X | X | X | X | X | X | X | X |
| B |  |  |  |  |  |  |  |  |

| 3. Clean up the work station, clear all fault codes |  |
| --- | --- |

### Questions

1.  What have you learned from this task?

___________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

| Unit |  | Task |  |
| --- | --- | --- | --- |
| 8 |  | 6 |  |
| Running 800 HO Engine |  |  |  |
|  |  | ATV | Side-by-Side |

# Fault Code Diagnosis

## Task Objective

At the completion of this task the technician will be able to properly perform various electrical circuit diagnostics by methods of dynamic testing and using input verses output strategies.  In addition, the student will be able to demonstrate how to perform a logical approach on an electrical diagnostic situation.

Materials

BUDS

MPI 2

Schematic

ECM diagnostic tool

Station binder and matching shop manual.

#### Repair Order Information

#### The customer states that engine won't start

Introduction

Now the diagnosis begins.  You need to apply a logical method of diagnosis.  For example, first check the schematic and check the wire colors and locations of the terminals and components.  Visually check for disconnected connectors or backed out terminals.  Ensure there is enough grip on the terminals.  Ensure the wires are in the correct positions in the connectors.  Test any power source and, or ground to the component at the component.  If any resistance values exist for the component, check those values at the component and if in specification, continue testing in the direction of the ECM connectors or ECM.  Remember, disconnecting or replacing the ECM should only be done as a last resort.  When it is necessary to disconnect the ECM, ensure to verify the continuity of the wires from the component to the ECM.  In conclusion, diagnosis is simply a process of eliminating what is good to determine what is bad.

Note: The ECM connector terminals are very delicate and therefore the ECM connections should never be removed unless absolutely necessary.  Perform as many tests as possible without disrupting the ECM connectors.

# Testing

Refer to the proper Station Binder section and the schematic and use a logical method for diagnosing this circuit malfunction.

# Procedures

####   Place the switches on the switch box in the following positions.

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | X | X | X | X | X |  | X | X |
| B |  |  |  |  |  | X |  |  |

|  |  |
| --- | --- |
| 1. Connect BUDS to the vehicle |  |
| 2. Install the DESS key in the vehicle. |  |
| 3. Read the BUDS data. |  |
| 4. Check the faults. |  |
| Record the faults and the descriptions here if there are any. |  |
| 5. Perform a logical method of diagnosis for this failure. |  |
| What needs to be repaired on the unit? Be specific list circuit numbers, color codes and any other pertinent information. |  |

####   Place the switches on the switch box in the following positions.

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | X | X | X | X | X | X | X | X |
| B |  |  |  |  |  |  |  |  |

| 6. Clean up the work station, clear all fault codes |  |
| --- | --- |

### Questions

1.  What have you learned from this task?

_____________________________________________________________________________________________________________________________________________________________________

Instructor sign off--  Go __________

| Unit |  | Task |  |
| --- | --- | --- | --- |
| 8 |  | 7 |  |
| Running 800 HO Engine |  |  |  |
|  |  | ATV | Side-by-Side |

# Fault Code Diagnosis

## Task Objective

At the completion of this task the technician will be able to properly perform various electrical circuit diagnostics by methods of dynamic testing and using input verses output strategies.  In addition, the student will be able to demonstrate how to perform a logical approach on an electrical diagnostic situation.

Materials

BUDS

MPI 2

Schematic

ECM diagnostic tool

Station binder and matching shop manual.

#### Repair Order Information

#### The customer states that there is no Cluster display anymore

Introduction

Now the diagnosis begins.  You need to apply a logical method of diagnosis.  For example, first check the schematic and check the wire colors and locations of the terminals and components.  Visually check for disconnected connectors or backed out terminals.  Ensure there is enough grip on the terminals.  Ensure the wires are in the correct positions in the connectors.  Test any power source and, or ground to the component at the component.  If any resistance values exist for the component, check those values at the component and if in specification, continue testing in the direction of the ECM connectors or ECM.  Remember, disconnecting or replacing the ECM should only be done as a last resort.  When it is necessary to disconnect the ECM, ensure to verify the continuity of the wires from the component to the ECM.  In conclusion, diagnosis is simply a process of eliminating what is good to determine what is bad.

Note: The ECM connector terminals are very delicate and therefore the ECM connections should never be removed unless absolutely necessary.  Perform as many tests as possible without disrupting the ECM connectors.

# Testing

Refer to the proper Station Binder section and the schematic and use a logical method for diagnosing this circuit malfunction.

# Procedures

####   Place the switches on the switch box in the following positions.

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | X | X | X | X | X | X |  | X |
| B |  |  |  |  |  |  | X |  |

|  |  |
| --- | --- |
| 1. Connect BUDS to the vehicle |  |
| 2. Install the DESS key in the vehicle. |  |
| 3. Read the BUDS data. |  |
| 4. Check the faults. |  |
| Record the faults and the descriptions here if there are any. |  |
| 5. Perform a logical method of diagnosis for this failure. |  |
| What needs to be repaired on the unit? Be specific list circuit numbers, color codes and any other pertinent information. |  |

####   Place the switches on the switch box in the following positions.

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | X | X | X | X | X | X | X | X |
| B |  |  |  |  |  |  |  |  |

| 6. Clean up the work station, clear all fault codes |  |
| --- | --- |

### Questions

1.  What have you learned from this task?

_____________________________________________________________________________________________________________________________________________________________________

Instructor sign off--  Go __________

| Unit |  | Task |  |
| --- | --- | --- | --- |
| 8 |  | 8 |  |
| Running 800 HO Engine |  |  |  |
|  |  | ATV | Side-by-Side |

# Fault Code Diagnosis

## Task Objective

At the completion of this task the technician will be able to properly perform various electrical circuit diagnostics by methods of dynamic testing and using input verses output strategies.  In addition, the student will be able to demonstrate how to perform a logical approach on an electrical diagnostic situation.

Materials

BUDS

MPI 2

Schematic

ECM diagnostic tool

Station binder and matching shop manual.

#### Repair Order Information

#### The customer states that there is an "E" in gear indicator on cluster when transmission is set to park(P) position

Introduction

Now the diagnosis begins.  You need to apply a logical method of diagnosis.  For example, first check the schematic and check the wire colors and locations of the terminals and components.  Visually check for disconnected connectors or backed out terminals.  Ensure there is enough grip on the terminals.  Ensure the wires are in the correct positions in the connectors.  Test any power source and, or ground to the component at the component.  If any resistance values exist for the component, check those values at the component and if in specification, continue testing in the direction of the ECM connectors or ECM.  Remember, disconnecting or replacing the ECM should only be done as a last resort.  When it is necessary to disconnect the ECM, ensure to verify the continuity of the wires from the component to the ECM.  In conclusion, diagnosis is simply a process of eliminating what is good to determine what is bad.

Note: The ECM connector terminals are very delicate and therefore the ECM connections should never be removed unless absolutely necessary.  Perform as many tests as possible without disrupting the ECM connectors.

# Testing

Refer to the proper Station Binder section and the schematic and use a logical method for diagnosing this circuit malfunction.

# Procedures

####   Place the switches on the switch box in the following positions.

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | X | X | X | X | X | X | X |  |
| B |  |  |  |  |  |  |  | X |

|  |  |
| --- | --- |
| 1. Connect BUDS to the vehicle |  |
| 2. Install the DESS key in the vehicle. |  |
| 3. Read the BUDS data. |  |
| 4. Check the faults. |  |
| Record the faults and the descriptions here if there are any. |  |
| 5. Perform a logical method of diagnosis for this failure. |  |
| What needs to be repaired on the unit? Be specific list circuit numbers, color codes and any other pertinent information. |  |

####   Place the switches on the switch box in the following positions.

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | X | X | X | X | X | X | X | X |
| B |  |  |  |  |  |  |  |  |

| 6. Clean up the work station, clear all fault codes |  |
| --- | --- |

### Questions

1.  What have you learned from this task?

____________________________________________________________________________________________________________________________________________________________________________________________________________________________

Instructor sign off--  Go __________