| Unit |  | Task |  |
| --- | --- | --- | --- |
| 10 |  | 14 |  |
| Outlander 800 MAX LTD |  |  |  |
|  |  | ATV | Side-by-Side |

### TPS Testing

## Task Objective

At the completion of this task the technician will be able to properly test a throttle position sensor circuit.

# Materials

Multi-meter

Wire diagram

Station Binder and corresponding service manual.

#### Repair order Information

#### The units Fuel Injection system seems to be suspicious, you are asked to check the TPS (Throttle Position Sensor)

#### Introduction

The inside view of BRP's ECM can be accessed while in Diagnostic/Calibration Mode.  It shows various faults and in some instances can indicate in which direction to troubleshoot.  However it is not an indicator that a component is absolutely at fault.  It merely indicates that there is an issue in the circuit that generated the fault code.

This is where the diagnosis begins.  You need to apply a logical method of diagnosis.  For example, first check the schematic and check the wire colors and locations of the terminals and components.  Visually check for disconnected connectors or backed out terminals.  Ensure there is enough grip on the terminal.  Ensure the wires are in the correct positions in the connector.  Ohm the component, first from the MPEM or ECM connector, then at the component.  Then verify the continuity of the wires from the component to the ECM.  Remember diagnosis is a process of eliminating what is good to determine what is bad.

TPS (Throttle Position Sensor)

The throttle position sensor senses how far the throttle is open.  The TP sensor is a variable resistor.  Otherwise, known as a voltage dividing circuit.  There are three wires going to the TP sensor.  Two wires are reference wires and one is the signal wire.  As illustrated in the following illustration the two reference wires are of a fixed voltage in this case we will say 5 volts one wire positive and one wire negative.  The voltage is passed along a fixed resistor inside the sensor.  The resistor consumes or drops the all the voltage across it length.  The third wire, the signal wire, is attached to a wiper that is in contact with and travels across the length of the fixed resistor as the throttle is opened or closed.  The ‘’signal’’ is the voltage read off of the wiper at any given location in its travel.

The adjustment of the throttle position is extremely important, if it is not correct the auto shift system will not function properly.  The throttle position sensor adjustment must be performed each time the sensor is loosened, removed and/or replaced.  This adjustment must also only be done after all carburetor settings and throttle adjustments have been done.  Refer to the proper shop manual for the correct specific procedure and setting specifications for each vehicle.

## TPS Testing / Input Sensor Testing

Refer to the Shop Manual

# Procedures

|  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| - Perform a TPS wear test as per shop manual |  |  |  |  |  |  |
| - Was the wear test successful? |  |  |  |  |  |  |
| YES |  | NO |  |  |  |  |
| - Perform a TPS resistance test as per shop manual |  |  |  |  |  |  |
| - What have you observed? (list your results here) ECM Connector; Throttle Idle Position; Throttle Wide Open Position. PIN; Resistance Ω at 20°C. |  |
| - Using the service manual schematic identify the TP sensor wires. |  |  |  |  |  |  |
| - What are the circuit terminal numbers at the ECM? |  |  |  |  |  |  |
| - What are the circuit terminal numbers at the TPS? |  |  |  |  |  |  |
| - List what circuit wire colors are in that circuit. |  |  |  |  |  |  |
| - Measure the TPS input voltage at the TPS connector according shop manual |  |  |  |  |  |  |
| - Measure the Volts between terminals #1 and battery ground on TPS connector |  |  |  |  |  |  |
| - What is the Volt value? |  |  |  |  |  |  |
| - Measure the Volts between terminals #2 and battery ground on TPS connector |  |  |  |  |  |  |
| - What is the Volt value? |  |  |  |  |  |  |
| - Measure the Volts between terminals #3 and battery ground on TPS connector |  |  |  |  |  |  |
| - What is the Volt value? |  |  |  |  |  |  |
| - Please list value you found suspicious below |  |  |  |  |  |  |

### Questions

1.  What have you learned from this task?

____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

- What type of circuit is a throttle position sensor?

________________________________________________________________________________________________________________________

Instructor sign off--  Go __________