| Station |  | Task |  |
| --- | --- | --- | --- |
| 8 |  | 1 |  |
| Water pump Rotary and Oil Seal Replacement |  |  |  |
|  |  | ATV | Side-by-Side |

V-Twin Water Pump Seals Removal and Installation

## Task Objective

At the completion of this task the technician will be able to properly remove and reassemble the water pump seals with the simplified procedure that avoids the water pump shaft removal thus eliminating the need to pull the engine out of the frame.

The procedure must be done carefully to avoid any damage on the shaft or on the crankcase.

# Materials

Outlander V-series crankcase with water pump elements.

Rotary seal

Oil seal

Shop manual V-Twin 2012 or up

# Tools

Rotary Seal Pusher Plate    529 036 130

4x M6 x 84 Screws              420 641 228

Seal Installer                       529 035 766

Slide Hammer

Extractor adaptor                 529 036 136

1xM8 Robust Flat Washer    503 183 100

1x M8 x 1.25 Nut                 233 281 414

2 x Flat screwdrivers

Driller

5 mm drill bit

Repair Order information

The customer complains there is a oil or coolant leak at the bottom part of the engine.

Initial inspection confirms the leak is on the leak indicator hole.

Procedures

Replace water pump rotary and oil seal according latest available Shop Manual

### Questions

1.  What have you learned from this task?

_______________________________________________________

_______________________________________________________

_______________________________________________________

2.  What is the pump shaft threads direction?

_______________________________________________________

3.  How many times do you have to install the pusher plate?

______________________________________________________________________

4.  What is the purpose of pushing the rotary seal in two steps??

______________________________________________________________________________________________________________

BE SURE TO STRAIGHTEN THE TOOLS AND THE WORKBENCH AREA

# Instructor sign off--  Go __________