| Unit |  | Task |  |
| --- | --- | --- | --- |
| 14 |  | 1 |  |
| Gear Box 1010 |  |  |  |
|  |  | ATV | Side-by-Side |

# Inspection and disassembly of the gearbox

## Task Objective

# -Once it has completed the task, the technician will be able to disassemble, inspect and assemble    the gearbox of a 1010 engine. The gearbox removed, the technician can make the game and  adjusting sproket

# Materials

Soft hammer

Big flat screwdriver

Torx 30

caliper

Micrometer

Loctite 5910

Soft rubber roller

Shop manual 219 100 452

Section 02 ENGINE, CVT AND GEARBOX

Subsection 11 (GEARBOX AND 4X4 COUPLING UNIT) 219100452

Repair order information

The gearbox does not snap and make noise

##### Introduction

- It is very important to make the inspection and reassembly of the gearbox. To do this, follow the instructions in the Repair Manual. Pay particular attention to the state of the lever alignment, the tree fork, the gear shift bush, shaft speed. In this transmission, the bevel gears transfer the power supplied by the transmission to the output shaft, which in turn transfers to the drive shaft and differential, and then to the wheels.

Procedure

|  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1-Perform the following steps and record the results. Although follow the manual procedures reparation |  |  |  |  |  |
| 2. Measure with micrometer n° 1 the diameter free pinion n° 2 values ​​range from 29,000 to 29,013 mm mm (1.1417 in. to 1.1422 in.) wear limit is 29,015 mm (1.1423 in.) What is the value ………………….. |  |  |  |  |  |
| the result is correct |  |  |  |  |  |
|  | yes |  | no |  |  |

| 2-Check shifting sleeves for worn inner splines and rounded or damaged engagement dogs 1. Inner splines 2. Engagement dogs -Measure the width of the groove engagement shift fork. The new value is 5.30 mm to 5.40 mm (.209 inch to .213 inch) wear limit is 5.50 mm (.217 inch). What is the value ………………….. The result is correct. Yes; no.. |  |
| --- | --- |

### 3-Shafts: Check bearing pin of reverse intermediate gear for wear or needle inprints: - Measure bearing pin outer diameter. The new value is 24.987 mm to 25.000 mm (.984 mm to 0984 po). Wear limit is 24.977 mm (.9833 po). What is the value …………………..

The result is correct

Yes; no.

| ### 4- Shafts: Measure countershaft bearing journals mag side. The new value is 19.977 mm to 19.990 mm. Service limit 19.970mm. Measure countershaft bearing journals cvt side. The new value is 24.977 mm to 24.990 mm. Service limit: 24.970 mm. A- Mag side B- Cvt side |  |
| --- | --- |
| ### The result it’s correct |  |
| ### yes; ### no. |  |

| 5- Check main shaft for wear: Measure free pinion bearing ( B ) the new value 24.987 mm to25.000 mm (.984 in to .984 in), service limit 16.976 mm (.668 in ). Measure bearing journal CVT / MAG side (A & C) the new value16.980 mm to 16.991 mm (.669 in to .669 in), service limit 16.976 mm (.668 in). A – Bearing journal MAG side B – Free pinion bearing C – Bearing journal CVT side T The result is correct. Yes; no.. |  |
| --- | --- |
| 6 – Shift Shaft: Check shift shaft for worn splines and gears. Check shift shaft spring fro damages. NOTICE : Do not disassemble shift drum - Check if shifting gates move easily on shift drum splines and check condition of springs - Check shift drum tracks for scouring or heavy wear, like rounded engagement slots. - Track for the low / reverse range gear shift fork - Track for the high range gear shift fork - Springs |  |

| 7- Shift Forks: Check both shift forks for visible damage. Check engagement rollers for wear and smooth movement. Measure shift fork claw thickness the new value 5.10 mm to 5.20 mm (.201 in to .205 in), service limit 5.00 mm (.197 in). 1- Micrometer A – Shift fork claw thickness The result is correct. Yes; no.. 8- 4x4 coupling Sleeve: check splines of coupling sleeve for wear or other damages 1- inspect splines Measure coupling sleeve groove width the new value 5.25 mm to 5.35 mm (.207 in to .211 in), service limit 5.50 mm (.217 in). The result is correct. Yes; no... |  |
| --- | --- |

| 9- Coupling Fork: Check both shift forks for visible damage. Check coupling fork claw thickness. Measure coupling fork claw thickness the new value 4.95 mm to 5.05 mm (.195 in to .199 in), service limit 4.80 mm (.189 in). 1- Coupling fork 2– Micrometer The result is correct. Yes; no.. |  |
| --- | --- |

### Questions

What have you learned from this task _______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

That happen if the coupling fork was bent coupling ____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

What happens if you spring the tree shift is damaged

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Instructor sign off--  Go __________