| Station | Task |
| --- | --- |
| 11 | 1 |
| Stationary Outlander Engine |  |

##### Drive Clutch Outlander

## Task Objective

At the completion of this task the technician will be able to properly disassemble, inspect and reassemble an ATV drive clutch using the proper BRP special tools.  He will also be able to clarify the areas of inspection and give rational for the inspection.  The technician will describe the function of the engine braking system as it applies to the drive clutch and list possible symptoms the vehicle may have if not functioning properly

# Materials

# Clutch tools

Crankshaft locking bolt

Torque wrench

Station binder

Shop manual 2012 Outlander/Renegade  800R / 1000  P/N 219 100 542

# Repair Order Information

The customer states that the unit will not shift when the engine is running.  It just grinds into gear.

Introduction

The Constantly Variable Transmission (CVT), also sometimes referred to as a variable pitch V-belt drive or torque-sensitive variable pitch V-belt drive has been employed on BRP snowmobiles since the introduction of the first Ski-Doo.  This drive system has also been adopted for use in several BRP  All Terrain Vehicles.

The main components of the Constantly Variable Transmission (CVT) consists of a drive pulley, (or “drive clutch”), a driven pulley (or “torque converter”, or “secondary pulley”) and a drive belt. The drive pulley is mounted on the crankshaft and turns at the same RPM as the crankshaft.  The CVT is used as a clutch, engaging and disengaging the transmission, and is also used to transfer the engine power to the drive train without having to change conventional gears.  The CVT can also feature an engine braking system for use during vehicle deceleration.

There are two modes that the CVT operate in.  One could be called the clutch mode and one could be called the transmission mode.  In the clutch mode, which is from idle speed to engagement speed, the crankshaft does not spin the drive pulley fast enough to engage the belt.  It acts as a disengaged clutch.  When the engine is accelerated and the system reaches a predetermined engagement speed, the clutch function ends and the belt engages and spins the driven pulley.  The pulleys become a continuously variable gear change transmission searching for the highest gear ratio that can be pulled at the engine’s given output.  The ratio changing is done by opening and closing a drive and driven pulley and forcing a fixed length drive belt to turn around different diameters at each pulley.  Depending on the BRP application, the ratio sometimes can vary from as low as 4 to 1 at engagement to as much as .70 to 1 in overdrive.  On the Outlander, the ratio at engagement is 2.9 to 1 and the final shift ratio in overdrive is .72 to 1.

Drive clutch not engaging belt			Drive clutch engaged and transmission shifting.

# The Drive Clutch

There are many variations in the construction of drive clutches.  However there are a few principles and components that the drive clutch needs to operate.  They include centrifugal force, rollers, a spring and ramps (in this case, on Outlanders the ramp could and is also called a flyweight and it is a combination weight and ramp).  In this section, we will examine how these items work together for the clutch to operate.

# Operation

The drive pulley is the RPM sensitive component of the CVT transmission.  The drive pulley is activated by flyweights, which are subjected to centrifugal force as they rotate.  At idle the drive belt rests between the open halves of the drive pulley.  The drive pulley remains open because of a spring located between the fixed and movable pulley halves (sheaves).  As the engine RPM is increased the flyweights are affected by the developing centrifugal force and then they begin to push against the sliding half pulley, moving it closer to the fixed half pulley.  As the pulley halves move closer together, pressure is applied to the sides of the drive belt and the belt begins to rotate with the pulleys.  This is known as belt engagement.  As the engine RPM is increased, the pressure continues to be increased on the sides of the drive belt.  Consequently, the belt is pushed or squeezed up the angled faces of the pulley halves.  When the belt travels up the pulley faces the ratio of the transmission changes or it begins to up-shift.  It should be noted here, that the further the belt travels up the drive pulley sheaves the belt tension increases in the driven pulley sheaves.  As the drive pulley up-shifts, the tension of the belt increases in the driven pulley sheaves causing the driven pulley to react to the increased belt tension and leverage created between the belt sides and the sheaves, this is also known as torque input.

In the style of clutch pictured below the rollers are fixed and the ramps (or flyweights) move against the rollers to achieve the same effect of converting the radial force to an axial force.

# Engine Braking

The design of most CVT systems disengages the drive belt when the engine returns to idle.  When this happens there is no way for the engines compression resistance to slow the vehicle down.  The vehicle can only be slowed by the application of the vehicles mechanical or hydraulic brake.  On some vehicles an engine braking system is provided to keep the vehicle from free-wheeling when the throttle is released or when going downhill.  According to the design of the CVT there are different ways to control the engine braking system.  What is common between these designs is that the belt always remains in contact with the drive and driven pulleys when the CVT is disengaged and there is some type of one-way clutch incorporated in the design.

When the clutch is disengaged, as long as the engine RPM is higher than the driven pulley shaft RPM the one-way clutch will be overrunning.  However, if the driven pulley shaft RPM begins to spin faster than the engine RPM, the one-way clutch will engage.  This will in turn allow the compression resistance of the engine to slow the vehicle down.  This is what is called, engine braking effect.

In this design engine braking device, as pictured in the following illustration, when the drive pulley is below engagement speed the belt falls onto a one way clutch in the center of the drive pulley.  The friction between the belt and this clutch is sufficient enough to engage the clutch.  As long as the engine RPM is higher than the driven pulley shaft RPM, the one-way clutch will overrun. However, if the driven pulley shaft begins to spin faster than the engine RPM, the one-way clutch will engage.  Inside the one way clutch is a ramp that a pin will follow.  When turned in the correct direction, the pin pulls a small shoulder affixed to the side of the clutch into contact with the belt.  The clutch then squeezes the belt between its shoulder and the side of the drive pulley transferring this force to the engines compression resistance.

NOTE: When removing the moving bushing, insure the two pins are not sprung out of position.

Refer to the station binder for the correct procedure and setting specifications.

# Procedures

|  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| - Remove the belt and the drive clutch as per the shop manual procedures. |  |  |  |  |  |
| - Inspect the engine braking device. |  |  |  |  |  |
| - Is the engine braking device in working order? |  |  |  |  |  |
| - YES |  | NO |  |  |  |
| - Inspect the rollers |  |  |  |  |  |
| - Are the rollers of the newest style? |  |  |  |  |  |
| - YES |  | NO |  |  |  |
| - Re-install the rollers and the drive clutch. |  |  |  |  |  |

### Questions

1.  What have you learned from this task?

______________________________________________________________________________________________________________

2.  What would happen if the engine braking device does not turn freely in one direction?

_______________________________________________________

3.  What is the torque spec of the drive clutch bolt?

_______________________________________________________

4. Regarding the bearings of the one way clutch. What is the difference between using the ATV 810 and the 1010 transmission?

_______________________________________________________

5. What is the meaning of the arrow at the governor cup

______________________________________________________________________________________________________________

6.   Why is there a difference between the V2 and 400 cc at the one way clutch

_______________________________________________________

Instructor sign off--  __________

| Station | Task |
| --- | --- |
| 11 | 2 |
| Stationary Outlander Engine |  |

##### Driven Clutch

## Task Objective

At the completion of this task the technician will be able to properly disassemble, inspect and reassemble an ATV driven clutch using the proper BRP special tools.  He will also be able to make clear, the areas of inspection, and give rational for the inspection.  The technician will describe the function of the engine braking system as it applies to the driven clutch and list possible symptoms the vehicle may have if not functioning properly.

# Materials

# Clutch tools

# Torque wrench

# Crankshaft locking bolt

Station binder

Shop manual 2012 Outlander/Renegade 800R / 1000  P/N 219 100 542

# Repair Order Information

Customer states the unit won’t up-shift completely.

Introduction

The Driven Pulley

Like the drive pulley, there are many variations in the construction of driven pulleys.  However there are a few principles and components that the driven pulley needs to operate.  They include belt tension, side-force, cam (helixes) and a spring.  In this section we will examine how these items work together for the driven pulley to operate.

# Operation

The driven pulley is the torque sensing component of the CVT transmission.  The driven pulley sliding half is influenced by a spring and cam arrangement that react to varying load conditions of the vehicles drive axle.  The driven pulley halves are held together by a spring located between the sliding half pulley (sheave) and a cam mounted on the end of the fixed pulley shaft.  In order for the belt to up-shift it has to overcome the side-force that is being applied to it by the driven pulley sheaves.  The tension of the belt in the driven pulley sheaves causes the driven pulley to rotate and begin to operate as soon as the drive pulley engages.  As the engine RPM increases, the belt rises between the sheaves of the drive pulley.  This increases the belt tension and the belt is pulled tighter into the sheaves of the secondary.  As the belt tension increases the "sides" of the belt have an increased leverage against the driven sheaves.  This allows the sheaves to separate and the belt to move lower between the two driven pulley halves.  The ratio between the drive and driven pulley changes as it begins to up-shift.  The combination of the spring tension, and the angle of the cam (or helix) work together to control the speed of the up-shift.  The leverage of the belt sides against the driven sheaves is what the driven pulley is reacting too, this is also called torque input.  Belt side angles, sheave side angles, belt durometer (or compound), belt tension, belt/sheave heat and the tuning of the driven pulley all have inputs on as to how the driven pulley reacts to the torque input.  The cooler the pulleys and the belt run, the more efficient the system is.  Another part not to be overlooked is the way the system back shifts.  If the rpms of the driven pulley is reduced because of less throttle input or higher loads on the vehicles drive axle, the belt tension will be reduced and the pulley system will backshift into a lower ratio.  The spring tension and the cam angle have an effect on how the driven pulley back shifts, as the driven pulley sheaves side force on the belt varies with engine RPM and drive axle load conditions.

It should be noted that as the driven pulley opens, the sliding half of the pulley is moving in three directions.  Forward with belt rotation, backward against the cam and laterally along the driven pulley shaft.  The forward travel is created by the driving direction of the belt.  The lateral movement occurs as the driven pulley up-shifts or down-shifts and the sheaves separate or come back together.  Since the slider shoes of the sliding half pulley are resting on the fixed cam, the lateral movement must also follow the contour of the cam angle surface.  As this happens, this lateral movement is also a backward movement.

While the drive pulley for the most part, controls the “up-shift” of the vehicle and maintains the engine operating RPM.  The driven pulley for the most part, controls the “backshift” or "down shifting" while the vehicles drive axle speed is reduced or put under heavier load.  Although keep in mind, the drive and driven pulleys calibrations work in conjunction of each other.  This is based on two matched and tuned calibrations that work together to achieve the best up-shift, downshift and efficiency for the application.  The overall efficiency of the CVT system is influenced greatly by the driven pulley.  The efficiency achieved by the driven pulley is maintained by the proper side force against the sides of the drive belt.  If there is too much side force, the engine through the drive pulley, will have to work harder to up-shift the belt.  If there is insufficient side pressure the belt may slip.  In either case, the belt will over heat.  Signs of overheating will be noticed by inspecting the sides of the belt for a glazed appearance, cords pulling out of the sides or top, or actual delamination of the belt.  Other signs of overheating or slippage could be pulleys that are “hot to the touch” or an excessive amount of belt dust scattered in the belly pan of the vehicle.  In some applications there is a belt deflection adjustment procedure and or belt height over the driven pulley specification.  Insure that the deflection and or belt height is adjusted correctly so the vehicle is in lowest gear ratio possible at engagement, but be sure that the deflection is not so tight, as to where the vehicle creeps at idle speed.  Always refer to the deflection specification for the application.  Some vehicles, such as the Outlander, do not have a belt deflection specification.  On the Outlander there is only a minimum belt width specification or service limit.  In any case, a worn out belt affects the engagement ratio all the way to full shift ratio.

The side force of the driven pulley sheaves on the belt sides and the tuned relationship between the drive and driven for the application, is carefully determined by the calibration engineers and should not need to be adjusted for the average rider.  However in some circumstances, spring pre-load adjustments, and an assortment of optional cams are available for the more experienced technician who might be qualified to understand the operation theory of the CVT system and wishes to experiment with changes.

Driven Pulley Cam (or Helix)

For a simple understanding of the driven pulley cam (or helix) refer to the illustrations below.  The illustrations below show a Ski-Doo cam as an example, but the up-shift and downshift principles of the cam angles are the same in all of BRP's applications.   There are variations in the cam angles available whether it is a straight cam, multi-angle cam or progressive angle cam.  The example below is a straight angle cam to make the example easier to understand.

Driven Pulley Spring

The driven pulley spring has much less affect on the driven pulley shifting sequence than does the cam.  The spring is needed, however, to keep the contact surface of the sliding sheave in contact with the cam and to provide enough side force when the belt is in the low ratio position to allow initial acceleration while the torque rises to a point where the torque sensing begins to take over.  The driven pulley spring tension has relatively low effect in the driven pulley shift pattern.  The driven pulley spring tension is changed only for relatively small changes in the driven pulley shift pattern.

Refer to the station binder containing the shop manual section 02, subsection 10 from the 2012 Outlanders shop manual for the correct procedure and setting specifications.

# Procedures

|  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - Remove the driven clutch as per the shop manual procedures. |  |  |  |  |  |  |  |  |
| - Inspect the cam helix for wear. |  |  |  |  |  |  |  |  |
| - What is the service limit of the cam? |  |  |  |  |  |  |  |  |
| - Check the torque gear for damages. |  |  |  |  |  |  |  |  |
| - What is the service limit on the teeth? |  |  |  |  |  |  |  |  |
| - Are the teeth within specification? |  |  |  |  |  |  |  |  |
| - YES |  |  | NO |  |  |  |  |  |
| - Re-install the driven clutch. |  |  |  |  |  |  |  |  |
| - What is the specification for drive belt width? |  |  |  |  |  |  |  |  |
| - Measure the drive belt. |  |  |  |  |  |  |  |  |
| - Is the drive belt within specification? |  |  |  |  |  |  |  |  |
| - What is the specification of the driven pulley end play? |  |  |  |  |  |  |  |  |
| - Install the belt. |  |  |  |  |  |  |  |  |
| - Remove the engine lock bolt. |  |  |  |  |  |  |  |  |
| - Is it nessesary to replace the Bolt of the driven Clutch everytime? |  |  |  |  |  |  |  |  |
| - YES |  |  |  | NO |  |  |  |  |

### Questions

1.  What have you learned from this task?

_____________________________________________________________________________________________________________________________________________________________________

2.  What is the torque spec of the driven clutch bolt?

_______________________________________________________

3.  What is the difference between the 800 and 1000 driven clutch spring?

_______________________________________________________

4.  What is to consider when installing the 1000 cc driven clutch on the countershaft?

_______________________________________________________

_______________________________________________________

_______________________________________________________

5.  What is the difference between the old and the new CVT cover?

_______________________________________________________

6. Are the covers interchangeable?

_______________________________________________________

Instructor sign off--  Go __________