| Station |  | Task |  |
| --- | --- | --- | --- |
| 10 |  | 1 |  |
| Differential Shimming |  |  |  |
|  |  | ATV | Side-by-Side |

#### Rear Differential Inspection and shimming procedure

## Task Objective

At the completion of this task the technician will be able to properly disassemble, inspect and reassemble rear differential on recreational vehicles.  The technician will be able to check the operation of the differential before and after disassembly, inspection and reassembly.  In addition, the technician will demonstrate the ability to replace the pinion gear, and select the shims to achieve proper backlash and preload.

# Materials

Rear differential with retaining plate

Backlash measurement Tool P/N 529 036 030

Spanner Socket P/N 529 035 649

Torque wrench 0,05 to 0,6 Nm

Caliper

Magnetic base stand with dial gauge

Propellershaft Bolt

T-40 Torx socket

3/8" Ratchet

13 mm nut

Spring scale 1 to 10 N

Differential Shim Kit P/N 703 500 548

Station Binder

Shop Manual 219 100 542 Outlander Renegade 2012

Repair Order information

The customer complains there is a grinding noise in the differential area.

Introduction

The disassembly, inspection and reassembly of the differential must be performed correctly. It is important to carefully follow the instructions in the shop manual.  Pay attention to details when inspecting the pinion gear, gears and bearings. When installing one case to another, hand-tighten the bolts and rotate everything before the torque is applied accurate measurements must be taken in order to select the proper shims for the preload .  The proper shims must be selected to achieve the proper backlash for quiet operation and long life.

Procedures

|  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| - Using the shop manual, record the specification for pinion gear backlash. from _____________ to ______________mm ( to in. ) |  |  |  |  |  |  |  |
| - Record the specification for the distance mark on the backlash tool tab. _____________mm ( in. ) from the center of the bolt (2) |  |  |  |  |  |  |  |
| - Position the dial indicator tip against the tab at a 90° angle and right on the previously scribed mark (2). Gently move the tool tab back and forth. Note the backlash result. Your result _____________ |  |  |  |  |  |  |  |
| - Rotate pinion gear ½ turn and recheck backlash and note the result. Your result ______________ |  |  |  |  |  |  |  |
| - Rotate pinion gear 1 turn and recheck backlash. Your result ______________ |  |  |  |  |  |  |  |
| - YES |  | NO |  | Is the backlash correct? |  |  |  |
| - Using the shop manual, record the specification for the differential preload. from _____________ to ______________Nm ( to lbf in. ) |  |  |  |  |  |  |  |
| - Using a needle torque wrench to measure the preload |  |  |  |  |  |  |  |
| - Install propeller shaft bolt into the pinion gear and measure the preload with the torque wrench or spring scale. Pay attention that the value of the spring scale is not the real torque. You have to convert the result. |  |  |  |  |  |  |  |
| 10. Your result ____________Nm |  |  |  |  |  |  |  |
| 11. YES |  | NO |  | Is the Preload correct? |  |  |  |

STOP!      Instructor Check Point      __________________

| 12. Split the final housing and measure the thickness of the backlash adjustment (1) shim and keep it for later installation !!!!!!! |  |
| --- | --- |
| 13. Your result __________________ |  |
| 14. Measure the thickness of the preload shim (1) and keep is for later installation!!!!!!!! |  |
| 15 Your result __________________ |  |
| 16. Unscrew the pinion nut by using the spanner socket. NOTE: The pinion nut is left hand threaded. |  |
| 17. Measure the thickness of the pinion gear shim.. |  |
| 18. Your result __________________ |  |

STOP!      Instructor Check Point      __________________

| 19. What is the recommended shim thickness for a preliminary adjustment? Backlash : _________ Preload: _________ Pinion gear: _________ |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 20. Reinstall the pinion gear |  |  |  |  |  |
| 21. Install the recommended shims and reassemble the final drive housing. |  |  |  |  |  |
| 22. What is the torque of the final drive housing screws? ____________ |  |  |  |  |  |
| 23. Check backlash specification |  |  |  |  |  |
| 24. Your result ____________ |  |  |  |  |  |
| 25. Check preload specification |  |  |  |  |  |
| 26. Your result ____________ |  |  |  |  |  |
| 27. What do you have to do if the torque of the preload is 0.59 Nm? __________________________________________________ __________________________________________________ ______________________________________________ 28. What do you have to do if the backlash is 0.30 mm ____________________________________________ ____________________________________________ ____________________________________________ |  |  |  |  |  |
| 29. If preload and/or backlash are out of specification. Replace shim(s) as described in shop manual to achieve correct specification. |  |  |  |  |  |
| 30. Thickness of preload shim _____________ |  |  |  |  |  |
| 31. Thickness of backlash shim ____________ |  |  |  |  |  |
| 32. YES |  | NO |  | Is the preload and backlash correct? |  |

STOP!      Instructor Check Point      __________________

| 33. Dissemble the final drive housing again and install the shims that were first installed. After installing the shims, hand tighten the final drive housing screws. |  |
| --- | --- |

### Questions

1.  What have you learned from this task?

_______________________________________________________

_______________________________________________________

_______________________________________________________

2.  What do you have to do if a new pinion nut is used.

_______________________________________________________

3.  What screw is used for the preload measurement

______________________________________________________________________________________________________________

_______________________________________________________

BE SURE TO STRAIGHTEN THE TOOLS AND THE WORKBENCH AREA

# Instructor sign off--  Go __________