| Unit |  | Task |  |
| --- | --- | --- | --- |
| 10 |  | 13 |  |
| Outlander MAX 800 LTD |  |  |  |
|  |  | ATV | Side-by-Side |

# Four-Stroke Engine Oil Pressure

## Task Objective

At the completion of this task the technician will be able to properly perform an oil pressure test.  Upon doing so the technician will be able to determine the condition of the engine oil pump, and engine plain bearings.  Also the technician will be able to demonstrate how to properly check the oil level on the engine.

# Materials

Station binder containing the shop manual

Oil pressure gauge 529 035 709 (Installed on engine)

Adapter hose P/N 529 035 652 (Installed on vehicle)

Repair Order Information

The customer states that the oil light is on and the engine sounds noisy.

#### Introduction

The life blood of the engine is the lubrication system.  If the lubrication system is not operating properly the engine will soon fail.  The primary purpose of the lubrication system is to reduce friction so to minimize wear, loss of power and engine performance.  It also helps to cool and clean the internal engine parts.  The oil is distributed throughout the engine starting at the pump sent through galleries to the crankshaft, camshaft bearings and other internal components.

(The bearings used on the DS 450 do not touch the other journal surface when the engine is running. All the components ride on a film of pressurized oil from the pump.)

The primary function of motor oil is to provide lubrication between moving metal parts in the engine. The friction caused by these parts when they move together creates high levels of heat that cause damage unless a lubricant is introduced to the mechanism.  A lubricant will smooth the surfaces and enable the parts to move freely against one another with reduced friction and, consequently, reduced wear and heat.

What would you do if a person came in with a complaint about the oil light being on or maybe with a knocking or tapping noise?  Well the first thing you do is check the oil level, and then you do an oil pressure test.  To obtain accurate oil pressure readings the following steps must be taken.  First, you locate the oil pressure tap.  In many cases the oil pressure sending unit is removed and its position is utilized.  Then install the oil pressure gauge.  After that you start the engine and record the reading.  Next, allow the engine to reach its normal operating temperature.  Then record the reading again.  Finally compare that reading to the specification listed in the shop manual.

###### Causes of Low Oil Pressure

Low oil pressure could be caused any of the following: A worn oil pump, the camshaft or crankshaft could have excessive bearing clearance.  There could be a broken or weak oil pressure regulator spring.  The pressure regulator could be stuck open.  You may have a clogged oil pickup screen, or possibly an air leak into the oil pump inlet.  Finally you could have excessively diluted oil.  Take an oil sample before you disassemble the engine.

In case you have high oil pressure it could be caused by either of the following, the pressure relief valve stuck closed, or the pressure relief valve spring tension could be too high.

# Four-Stroke Engine Oil Pressure Testing

Refer to the proper Shop Manual in the station binder for the correct oil pressure testing procedure and specifications.

# Procedures

|  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| - Follow the following steps and record the information below. |  |  |  |  |  |
| - List the oil pressure specifications from the gauge that is on the running station. For the purposes of this school the gauges are already installed for you. Refer to the oil pressure gauge installation location as per the shop manual. |  |  |  |  |  |
| - Where is oil pressure gauge installed on the engine? |  |  |  |  |  |
| - What is the oil pressure specification? @ Idle? –_________ Do not run the engine! |  |  |  |  |  |
| - What is the oil pressure specification? @ 5000 RPM –_________ Do not run the engine! |  |  |  |  |  |
| - Start the engine and observe the oil pressure – Remember no more than 5000 rpm! Record gauge values here – @ Idle RPM =_________@ 5000RPM =_________ |  |  |  |  |  |
| - Is the oil pressure within the specifications? |  |  |  |  |  |
| - YES |  | NO |  |  |  |

### Questions

1.  What have you learned from this task?

___________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

- List six causes of low oil pressure.

- ___________________________________________

- ___________________________________________

- ___________________________________________

- ___________________________________________

- ___________________________________________

- ___________________________________________

### Instructor sign off--  Go __________