| Station |  | Task |  |
| --- | --- | --- | --- |
| 13 |  | 1 |  |
| Stationary V-1010 |  |  |  |
|  |  | SSV | ATV |

# 4-Stroke Cam Timing Procedures – V-1010

## Task Objective

At the completion of this task the technician will be able to properly lock the engine crankshaft and camshaft.  He will also be able to perform the cam timing procedure in the correct sequence.

# Materials

# V1010 4-TEC engine.

# Crankshaft locking tool: 529035617

# Camshaft locking tool: 5290035926

# Camshaft timing tool: 529036231

Crankshaft TDC position tool: 529036201

1/4", 1/2" drive ratchets

13mm and 15mm and 8mm wrench

Small flat blade screw driver

1/2" 14mm allen sockets, spark plug socket 16mm

½" extension and toggle

4 and 5 mm allen socket (1/4 drive)

T 30 socket

1/4"drive x 4" long extension

Feeler gauge

# Rags

Wire

# Shop manual 219100542

# Introduction

Camshaft timing is obviously crucial to proper engine performance.  Proper camshaft timing ensures that the intake and exhaust valves are opened and closed at precisely the right time.  If the camshaft timing is incorrect, the valves may open and close too soon or too late.  This will result in poor engine performance, (if the engine is able to start at all).  If valve timing is seriously out of synchronization with the rotation of the crankshaft, the valves may open when the piston is approaching TDC, and the piston will collide with the valves.  This will result in bent valves, which will no longer seat or seal properly.  Serious piston damage can also result.  In either case, once the valves are bent, the engine will no longer run.

Careful and accurate cam timing is particularly important in dual over-head cam (DOHC) engines.  Since DOHC engines use separate intake and exhaust cams, it is crucial that the position of both cams be carefully synchronized with the crankshaft.  Otherwise, not only can the valves collide with the pistons but the intake and exhaust valves could collide with one another.

# Camshaft Timing

# Refer to the appropriate shop manual as listed in the Materials section.  One of the first steps in performing any procedure is to locate the information and read it prior to performing the procedures.  The technician should ensure all necessary tools are available.  The shop manual is the best resource for information, specifications and step by step instructions pertaining to all aspects of the vehicle.  In summary, the importance of the shop manual can not be overstated and its use will always be the required first step of any procedure.

# Procedures

Engine preparation:

- Remove spark plugs

- Remove valve covers

- Remove CPS

|  |  |
| --- | --- |
| - Follow the procedures provided in this task sheet and the shop manual to complete the repair and record the following information. # Lock cylinder number 2 at TDC. Make sure the locking bolt engages the groove in the crankshaft. Note: Cylinder number 1 cannot be locked at TDC using the crankshaft locking tool. # Note (Do not use the Crankshaft TDC position tool: 529036201 to lock the crankshaft). |  |
| - Remove the chain tensioner for cylinder number 2. |  |
| - Use the camshaft locking tool, lock the camshaft at TDC. # Note (Do not use the camshaft timing tool: 529036231 to lock the camshaft). |  |
| - Why must the camshaft locking tool be used? |  |
| - Remove the camshaft timing gear. Make sure to use a piece of wire to secure the timing chain from dropping into the engine bottom half. |  |
| - Now rotate the crankshaft counterclockwise (approximately 280°) until the marks on the magneto flywheel number 1 and the magneto cover, are aligned. Extreme care needs to be taken so that the timing chain for cylinder number 2 does not get hung up or stretched. |  |
| - Remove the chain tensioner for cylinder number 1. |  |
| - Use the camshaft locking tool and lock the camshaft at TDC. Once locked, remove the gear as you did in step # 6. Remember to use a wire to secure the timing chain. |  |
| - Remove the camshaft retainers, then the rocker shafts and the rocker arms from each cylinder. |  |
| - Remove both camshafts. |  |
| - Carefully rotate the crankshaft clockwise and lock cylinder number 2 at TDC using the crankshaft locking bolt. Both timing chains need to be monitored closely so they do not jump off their respective crank gear, hang up or drop into the bottom end. |  |

STOP! 	Instructor Check Point	____________

| 13. Re-install the camshaft for cylinder number 2. |  |
| --- | --- |
| 14. Re-install the rocker arms and shafts for cylinder number 2. |  |
| - Set cylinder 2 to ignition TDC |  |
| - Install crankshaft TDC position tool. Make sure to match the teeth on the tool with the magneto rotor |  |
| 17. Install camshaft timing tool on cylinder head |  |
| # 18. Note: Do not use the Camshaft locking tool: 5290035926 and ensure the moving blade contacts entirely with the flat portion of the camshaft flange. |  |
| 19. Install crankshaft timing gear with timing chain on camshaft. Don't tight the retaining bolts at this time. (The gear should be able to rotate freely during the installation of the tensioner) |  |

| - Install the timing chain tensioner for cylinder number 2 paying close attention to the procedure in the shop manual (page 138) |  |
| --- | --- |
| 21. Install and tighten the timing gear retaining screws |  |
| 23. Remove the crankshaft position tool and turn the engine counterclockwise as viewed from the magneto side of the engine (approximately 280°) until cylinder number 1 is at TDC. Note: With cylinder number 1 at TDC, cylinder number 2 should appear to have the timing marks in parallel to the table. They are inversely proportional. |  |
| 24. Install crankshaft TDC position tool and repeat the procedures described for the cylinder #2 |  |

| 25. Remove the camshaft and crankcase position tools and make sure the engine rotates freely. 26. Perform the valve adjustment as per shop manual |  |
| --- | --- |
| 26. Install any removed part. |  |
| 33. Clean up the workstation. |  |

### Questions

1.  What have you learned from this task?

______________________________________________________________________________________________________________

_______________________________________________________

2. What would happen if this procedure where not done correctly?

_____________________________________________________________________________________________________________________________________________________________________

- Why the new camshaft timing gears have slotted holes?

_____________________________________________________________________________________________________________________________________________________________________

- Why the crankshaft is turned counterclockwise on step 23? Is there any way to time the engine correctly turning the engine clockwise?

_____________________________________________________________________________________________________________________________________________________________________

Instructor sign off--  Go __________