| Unit |  | Task |  |
| --- | --- | --- | --- |
| 10 |  | 1 |  |
| ATV DPS |  |  |  |
|  |  | ATV | SSV |

# ATV DPS (Dynamic Power Steering)

## Task Objective

At the completion of this task the technician will be able to properly utilize the troubleshooting flow charts and BUDS for proper DPS diagnosis.  In addition he will be able to give a brief explanation about the main components and their functions in a DPS system.

# Materials

Computer

BUDS

Task sheet

Repair Order Information

Customer states "check DPS" scrolled on cluster or no assistance is present and wants DPS checked

##### Introduction

In overview, the Dynamic Power Steering (DPS) provides a computer controlled, variable power assist, achieved by an electric motor to optimize the amount of steering input required by the rider.  The DPS (Dynamic Power Steering) unit is NOT a pump. It hydraulically pumps nothing. The DPS motor does not spin, it rotates slowly or not at all most of the time providing slight assist to the steering effort of the rider. It only rotates 6 revolutions total from full lock to full lock of the steering.

# ATV DPS (Dynamic Power Steering)

Station overview

This station was created to help students understand the BRP Dynamic Power Steering system, how it works, and how it is self protected.

Shocks in place of driving related resistance

These two High Pressure Gas (HPG) shocks were specially crafted to provide high resistance when turning the handle bars to the left and to the right. They simulate varying terrain conditions while driving an ATV with DPS.

Procedures

####   Place the switches on the switch box in the following positions.

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | X | X | X | X | X | X | X | X |
| B |  |  |  |  |  |  |  |  |

|  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| - Turn ignition ON, read out data with B.U.D.S and go to the DPS monitoring page. Turn handle bars back and forth several times. Note the amount of effort required: |  |  |  |  |  |
| - Start the engine |  |  |  |  |  |
| - Turn handlebars back and forth several times. |  |  |  |  |  |
| - Is the steering easier? |  |  |  |  |  |
| YES |  | NO |  |  |  |
| - Note: The amount of "Output Current" indicates visually the level of assist that the DPS unit exerts into the steering system to help the operator steer or to reduce "feedback" felt by the rider due to rough terrain |  |  |  |  |  |
| - What is the maximum number of Amps now observed when turning left and right? _____________. |  |  |  |  |  |
| - Now repeat the procedures while observing the driver input in torque versus the assist level in Amps. Note your conclusions observing both gauges: ____________________________________________________________________ _____________________________________________________________________________________________________________________________________________________________________________________________________________________ |  |  |  |  |  |
| 32. Turn the key to off. |  |  |  |  |  |

The objective of this next section is to observe the effects of having the Steering Torque Offset set incorrectly.

| - Turn the key and stop switch to the run positions, do NOT start the engine |  |
| --- | --- |
| - Launch BUDS and read the MPEM. |  |
| - Go to the Settings Page, and choose DPS. |  |
| - Observing the Torque Gauge, and apply enough torque to the handlebars to move the needle to about 6 Nm. |  |
| - While holding the torque at 6 Nm Click Reset Torque Offset and confirm the reset. |  |
| - Let go of the handlebars. |  |
| - What is the Torque Offset? _________________. |  |
| - If this reading were observed with the vehicle at rest with no torque applied to the handlebars, would it be an acceptable condition? _________________________. |  |
| - Now please start the engine |  |
| - As the machine starts observe the conditions. |  |
| - Let go of the handlebars. |  |
| - What do the handlebars do? _______________________________________. |  |
| - As you move the handlebars can you feel the effects of an improperly set Torque Offset? ________________. |  |
| - What can you observe about the amperage draw with the Torque Offset not set incorrectly? ______________________________________________________________________________________________________________________________________________ |  |
| - Turn the key OFF |  |
| - After shut down, turn on ignition again, do NOT start the engine |  |
| - Properly Zero the Torque Offset. |  |
| - Note: If you continually see a message that states the "engine must be stopped, or the conditions are not correct for this action" You must close BUDS and re-launch it. |  |
| - When the Offset is set correctly, start the training machine. |  |
| - Do the handlebars stay in a neutral position? ___________________________. |  |
| - Turn the Key off and Exit BUDS. |  |

The objective of this next section is to demonstrate the proper way to shim the steering column on a DPS equipped ATV.

| - Refer to the 2010 DPS/ACS Technical Manual page 20 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| - Loosen the bolts securing the upper steering column support half bushings until they are finger tight. |  |  |  |  |  |
| - Remove the shim at the rear of the steering column support. |  |  |  |  |  |
| - Measure the support and record its thickness. - ______________ |  |  |  |  |  |
| - Measure the distance of the steering column to support according to the shop manual and record that distance. - _________________ |  |  |  |  |  |
| - What is the correct shim required for the suport according to the shop manual? - ______________ |  |  |  |  |  |
| - Was the correct shim installed in the vehicle? |  |  |  |  |  |
| YES |  | NO |  |  |  |
| - Reinstall the shim at the rear of the steering column. |  |  |  |  |  |
| - Tighten the support column bolts. |  |  |  |  |  |
| - Clean up the work station. |  |  |  |  |  |

### Questions

1.  What have you learned from this task?

____________________________________________________________________________________________________________________________________________________________________________________________________________________________

- What could happen if the incorrect shim were used in the steering column support half bushings?

______________________________________________________

- What needs to be done if a DPS unit is replaced?  (Circle all that are correct)

Install a new .FC file

Reset the torque sensor

Select the correct shim for the steering column support

Perform a front end alignment

Rotate the tires

### Instructor sign off--  Go __________

| Unit |  | Task |  |
| --- | --- | --- | --- |
| 10 |  | 2 |  |
| ATV DPS |  |  |  |
|  |  | ATV | SSV |

#### Dynamic testing

## Task Objective

At the completion of this task the technician will be able to properly diagnose an improperly working DPS System

# Materials

Multi-meter

Brain

Back-probe tools

Test light

BUDS

Schematic

Repair Order information

The customer complains that the Dynamic Power Steering does not provide assist.

Introduction

This is where the diagnosis begins.  You need to apply a logical method of diagnosis.  For example, you should think of what the repair order complaint is and possible causes for it.  Remember the basics first:  Does it start?  Does it run on all the cylinders?  How is the compression?  How about fuel pressure?  Are there any fault codes?  Does all the monitored data look correct?  If you determine it is an electrical issue, first check the schematic and check the wire colors and locations of the terminals and components.  Visually check for disconnected connectors or backed out terminals.  Ensure there is enough grip on the terminal.  Ensure the wires are in the correct positions in the connector.  Ohm the component, first from the MPEM or ECM connector, then at the component.  Then verify the continuity of the wires from the component to the ECM.

Use the tools you have at hand, the most important ones being the knowledge that you have and the thought process you use to narrow the problem down to its root cause.  Remember diagnosis is a process of eliminating what is good to determine what is bad.

Procedures

####   Place the switches on the switch box in the following positions.

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A |  | X | X | X | X | X | X | X |
| B | X |  |  |  |  |  |  |  |

|  |  |
| --- | --- |
| - What trouble shooting flow chart needs to be used for this complain? |  |

| What needs to be repaired on the unit? Be specific list circuit numbers, color codes and any other pertinent information. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| - Does the flow chart guide you to the cause of the issue? |  |  |  |  |  |
| YES |  | NO |  |  |  |

####   Place the switches on the switch box in the following positions.

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | X | X | X | X | X | X | X | X |
| B |  |  |  |  |  |  |  |  |

| - Clear any codes, clean up work station and disconnect BUDS. |  |
| --- | --- |

### Questions

1.  What have you learned from this task?

____________________________________________________________________________________________________________________________________________________________________________________________________________________________

# Instructor sign off--  Go __________

| Unit |  | Task |  |
| --- | --- | --- | --- |
| 10 |  | 2 |  |
| ATV DPS |  |  |  |
|  |  | ATV | SSV |

#### Dynamic testing

## Task Objective

At the completion of this task the technician will be able to properly diagnose an improperly working DPS System

# Materials

Multi-meter

Brain

Back-probe tools

Test light

BUDS

Schematic

Repair Order information

The customer complains that the Dynamic Power Steering does not provide assist.

Introduction

This is where the diagnosis begins.  You need to apply a logical method of diagnosis.  For example, you should think of what the repair order complaint is and possible causes for it.  Remember the basics first:  Does it start?  Does it run on all the cylinders?  How is the compression?  How about fuel pressure?  Are there any fault codes?  Does all the monitored data look correct?  If you determine it is an electrical issue, first check the schematic and check the wire colors and locations of the terminals and components.  Visually check for disconnected connectors or backed out terminals.  Ensure there is enough grip on the terminal.  Ensure the wires are in the correct positions in the connector.  Ohm the component, first from the MPEM or ECM connector, then at the component.  Then verify the continuity of the wires from the component to the ECM.

Use the tools you have at hand, the most important ones being the knowledge that you have and the thought process you use to narrow the problem down to its root cause.  Remember diagnosis is a process of eliminating what is good to determine what is bad.

Procedures

####   Place the switches on the switch box in the following positions.

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | X |  | X | X | X | X | X | X |
| B |  | X |  |  |  |  |  |  |

|  |  |
| --- | --- |
| - What trouble shooting flow chart needs to be used for this complain? |  |

| What needs to be repaired on the unit? Be specific list circuit numbers, color codes and any other pertinent information. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| - Does the flow chart guide you to the cause of the issue? |  |  |  |  |  |
| YES |  | NO |  |  |  |

####   Place the switches on the switch box in the following positions.

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | X | X | X | X | X | X | X | X |
| B |  |  |  |  |  |  |  |  |

| - Clear any codes, clean up work station and disconnect BUDS. |  |
| --- | --- |

### Questions

1.  What have you learned from this task?

___________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

# Instructor sign off--  Go __________

| Unit |  | Task |  |
| --- | --- | --- | --- |
| 10 |  | 3 |  |
| ATV DPS |  |  |  |
|  |  | ATV | SSV |

#### Dynamic testing

## Task Objective

At the completion of this task the technician will be able to properly diagnose an improperly working DPS System

# Materials

Multi-meter

Brain

Back-probe tools

Test light

BUDS

Schematic

Repair Order information

The customer complains that the Dynamic Power Steering does not provide different assist modes anymore.

Introduction

This is where the diagnosis begins.  You need to apply a logical method of diagnosis.  For example, you should think of what the repair order complaint is and possible causes for it.  Remember the basics first:  Does it start?  Does it run on all the cylinders?  How is the compression?  How about fuel pressure?  Are there any fault codes?  Does all the monitored data look correct?  If you determine it is an electrical issue, first check the schematic and check the wire colors and locations of the terminals and components.  Visually check for disconnected connectors or backed out terminals.  Ensure there is enough grip on the terminal.  Ensure the wires are in the correct positions in the connector.  Ohm the component, first from the MPEM or ECM connector, then at the component.  Then verify the continuity of the wires from the component to the ECM.

Use the tools you have at hand, the most important ones being the knowledge that you have and the thought process you use to narrow the problem down to its root cause.  Remember diagnosis is a process of eliminating what is good to determine what is bad.

Procedures

####   Place the switches on the switch box in the following positions.

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | X | X |  | X | X | X | X | X |
| B |  |  | X |  |  |  |  |  |

|  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| - Turn on the ignition, read B.U.D.S. an go to the DPS monitoring page |  |  |  |  |  |
| - Change the DPS assist mode between Min and Max |  |  |  |  |  |
| - Can you change the Mode? |  |  |  |  |  |
| YES |  | NO |  |  |  |
| - What needs to be repaired on the unit? Be specific list circuit numbers, color codes and any other pertinent information. |  |  |  |  |  |

####   Place the switches on the switch box in the following positions.

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | X | X | X | X | X | X | X | X |
| B |  |  |  |  |  |  |  |  |

| - Turn on the ignition, read B.U.D.S. an go to the DPS monitoring page |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| - Change the DPS assist mode between Min and Max |  |  |  |  |  |
| - Can you change the Mode? |  |  |  |  |  |
| YES |  | NO |  |  |  |
| - Please start the engine and go to the B.U.D.S. monitoring page |  |  |  |  |  |
| - Change the DPS assist mode between Min and Max and observe the Amperage draw |  |  |  |  |  |
| - List the max Amp in both modes: Min: Max: |  |  |  |  |  |

| - Clear any codes, clean up work station and disconnect BUDS. |  |
| --- | --- |

### Questions

1.  What have you learned from this task?

___________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

2.  Please tell us another situation when the change of DPS Mode is not applicable?

___________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

# Instructor sign off--  Go __________

| Unit |  | Task |  |
| --- | --- | --- | --- |
| 10 |  | 4 |  |
| ATV DPS |  |  |  |
|  |  | ATV | SSV |

#### Dynamic testing

## Task Objective

At the completion of this task the technician will be able to properly diagnose an improperly running DSP System

# Materials

Multi-meter

Brain

Back-probe tools

BUDS

Schematic

Repair Order information

The customer complains that the Dynamic Power Steering does not feel the same anymore since several days, there might be no or limited assistance only.

Introduction

This is where the diagnosis begins.  You need to apply a logical method of diagnosis.  For example, you should think of what the repair order complaint is and possible causes for it.  Remember the basics first:  Does it start?  Does it run on all the cylinders?  How is the compression?  How about fuel pressure?  Are there any fault codes?  Does all the monitored data look correct?  If you determine it is an electrical issue, first check the schematic and check the wire colors and locations of the terminals and components.  Visually check for disconnected connectors or backed out terminals.  Ensure there is enough grip on the terminal.  Ensure the wires are in the correct positions in the connector.  Ohm the component, first from the MPEM or ECM connector, then at the component.  Then verify the continuity of the wires from the component to the ECM.

Use the tools you have at hand, the most important ones being the knowledge that you have and the thought process you use to narrow the problem down to its root cause.  Remember diagnosis is a process of eliminating what is good to determine what is bad.

Procedures

####   Place the switches on the switch box in the following positions.

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | X | X | X |  | X | X | X | X |
| B |  |  |  | X |  |  |  |  |

|  |  |
| --- | --- |
| - What trouble shooting flow chart needs to be used for this complain? |  |

| What needs to be repaired on the unit? Be specific list circuit numbers, color codes and any other pertinent information. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| - Does the flow chart guide you to the cause of the issue? |  |  |  |  |  |
| YES |  | NO |  |  |  |

####   Place the switches on the switch box in the following positions.

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | X | X | X | X | X | X | X | X |
| B |  |  |  |  |  |  |  |  |

| - Clear any codes, clean up work station and disconnect BUDS. |  |
| --- | --- |

### Questions

1.  What have you learned from this task?

____________________________________________________________________________________________________________________________________________________________________________________________________________________________

# Instructor sign off--  Go __________

What you have observed is the Protection Mode, also known as de-rating. If the charging system is unable to maintain 13 volts the DPS system reduces the level of assist by decreasing the amperage applied to the DPS motor. By doing this, some assist is available to "help" the rider steer the vehicle while protecting the battery voltage to drop further. You can see by the chart that over 2000 RPM when the battery drops from 13 to 12 volts, there is a 60% reduction in steering assist. This is what you have observed and felt. Notice that as the battery voltage continues to drop, the level of assist also decreases as designed into the logic of the system.

|  | 1000 RPM | 2000 RPM | 4000 RPM | 5000 RPM | 6000 RPM | 7000 RPM |
| --- | --- | --- | --- | --- | --- | --- |
| 8 V | 5% | 8% | 11% | 14% | 17% | 20% |
| 9 V | 10% | 14% | 18% | 22% | 26% | 30% |
| 10 V | 15% | 20% | 25% | 30% | 35% | 40% |
| 11 V | 25% | 30% | 35% | 40% | 45% | 50% |
| 12 V | 35% | 40% | 45% | 50% | 55% | 60% |
| 13 V | 100% | 100% | 100% | 100% | 100% | 100% |

Extra Information:

3rd DPS Mode Electronic Module

Only available with the apache track kit WT systems.  The 3rd DPS mode, when installed with the apache WT system optimizes power steering performance.  It is a module for the dynamic power steering maximizing steering agility at lower speeds while providing firmer steering at cruising speeds. In addition, it provides a speed correction factor for the speedometer.  It is installed on the 6 pin DLC (Diagnostic Link Connector) and communicates via CAN (Controller Area Network) to the DPS cluster and ECM.  Once installed, the module will only be functional for the vehicle it was initially configured to.

Transferring a Wide Track Kit from one ATV to another

You may encounter issues when transferring a Wide Track Kit from one ATV to another if the track kit is equipped with the optional 3rd Mode DPS Module.  When transferred to the new vehicle, the 3rd Mode DPS Module must be reinitialized.

The steps to do so are:

1. Connect BUDS to the vehicle. Click on "read data"

2. Go to "Module menu", select "CCK", then "3rd DPS mode", then "Information"

3. In the window that appears, click on the "Refresh" button

4. Turn the key off and wait for the vehicle to completely power down which takes about 30 seconds.

For complete instructions on installing and configuring the 3rd Mode DPS Module, refer to Instruction Sheet P/N 487 800 454.

Notice: Once installed, the module will only be functional for the vehicle it was initially configured to.

DPS Software Updates

When replacing a DPS unit a (.FC file) must be installed.  It is the calibration program that allows the DPS to be properly recognized and function as it should.  If the program is not installed the Torque Sensor will not be able to be reset and DPS code P0601 (CRC fault) will be set.

The reason there are different .FC files is that the file must recognize the correct BRP assembly number, which changes with DPS production.  As the assembly number changes refer to BOSSWeb to obtain the correct .FC file.