| Station |  | Task |  |
| --- | --- | --- | --- |
| 15 |  | 1 |  |
| Wiring diagram exercise |  |  |  |
|  |  | ATV | SSV |

# ATV Wiring Diagram

## Task Objective

At the completion of this task the technician will be able to properly identify various circuits in ATV wire schematics.  The technician will also be able to explain the meaning of each of the different code identifications within the schematic.  Then the technician will be able to explain what would be the best approach to troubleshooting an identified circuit.

##### Materials

Outlander 800 general system schematic

5 colored pens

- Yellow

- Green

- Orange

- Blue

- Red

- Brown

- Purple

Task sheet

Station binder containing

- Shop Manual

# UNDERSTANDING ATV WIRING DIAGRAMS

Using the supplied diagrams, identify the following:

- On the Outlander diagram, use a blue highlighter and circle the ignition key for the (lights), the ignition key for (DESS) and the ignition key for (ON).

- Using a red marker trace out the entire 12 Volts positive supply wire from the battery to the main relay (R2).

- What is the wire color that supplies 12 Volts positive to the main relay? ___________________

- What terminal numbers are supplied 12 volts positive power at the main relay? _______________and ________________

- What is, the fuse number and how many amps is the fuse that protects the 12 volt positive power to the main relay?

Fuse number = ________Amps = _______

- When the ignition switch is turned to the on position, a ground is supplied to the low current side of the main relay.   With the green marker trace out the 12 volt negative (ground path) from the relay to the negative side of the battery.

- How many times does the wire color change on the ground path to the battery from the main relay? ___________

- List the wire colors of the ground path of the main relay low current side to the battery.

A. _______ B._______ C.  _______ D. _______ E. _______ F. ________

- With the yellow marker trace the 12 volts positive supply from the high current side of the main relay to the ECM.

- What is the terminal number at the ECM that the 12 volt positive enters the ECM? ___________

- Using a green marker trace out all of the 12 volt negative (ground) wires, from the battery to the ECM.

- What are the terminal location numbers of the ground wires at the ECM? ____________________________________________________________

- What would happen if the PF1 one-way diode (D1) was installed backwards in the fuse block?______________________________________________

- Using an orange marker trace out the 12 volts positive supply wires from the CV speed sensor and the CAPS (Cam Shaft Sensor) to the source of the 12 volts positive power for those two sensors.

- What component supplies the 12 volts positive power to those two sensors? ______________________________________________________________

- What is, the fuse number and how many amps is the fuse that protects the 12 volt positive power to cam shaft sensor?

Fuse number = ______Amps = ______

- List all the components that would not function if fuse F4 was open?

________________________________________________________________________________________________________________________________

- What are the terminal locations of the 12 volt negative (ground) for the MAPTS at the ECM and at the MAPTS sensor? _______________________

- What is the terminal location at the ECM for the signal wire of the air pressure sensor? _____________________________________________

- What zone number and zone location(coordinates) on the vehicle is the sub-transmission switch located?  _______________

- What is the maximum number of amps allowed on a # 14 AWG wire? _____

- Identify the following schematic symbols.

- What component activates the low current side of the Accessories (R3) relay?  What is the wire color and what is the terminal number at the ECM?  ___________________________________________________________

- A customer has a complaint of a check engine light and a code P0600 (communication problem – instrument cluster message missing).  What color wires should be checked for continuity and what are the terminal locations to check between and at the cluster and the ECM for the communication wires?

Color = _______ and ________ Terminal locations at Cluster =_______ and ________ and at ECM = __________ and __________.

### Instructor sign off--  Go __________

| Station |  | Task |  |
| --- | --- | --- | --- |
| 15 |  | 2 |  |
| Wiring diagram exercise |  |  |  |
|  |  | ATV | SSV |

# SSV Wiring Diagram

## Task Objective

At the completion of this task the technician will be able to properly identify various circuits in SSV wire schematics.  The technician will also be able to explain the meaning of each of the different code identifications within the schematic.  Then the technician will be able to explain what would be the best approach to troubleshooting an identified circuit.

##### Materials

SSV general system schematic

5 colored pens

- Yellow

- Green

- Orange

- Blue

- Red

Task sheet

Station binder containing

- Shop Manual

# UNDERSTANDING ATV WIRING DIAGRAMS

Commander Schematic Exercise

- Using a BLUE marker circle the battery, the main relay, the ignition switch, the diagnostic connector, the speedometer, the accessories relay, the lights relay and the fan relay.

- What number is the main relay? ______________

- Using a RED marker and starting from the battery trace the line through main fuse F1 to the ignition switch, the low and high current side of the main relay ,, the speedometer, and the diagnostic connector.

- What number fuse protects the ignition switch and how many amps is it?

Fuse number = _________  Amps = __________

- Using a RED marker and starting from the battery trace the line through the fan / accessory fuse F2 to the lights relay, the accessories relay and the fan relay.

- What side of the relays is supplied current from the 40 amp fan accessories fuse number (F2). (The high current side or the low current side? __________________________________________________

- Using a GREEN marker and starting from ground nut connection 5 BN 1 trace the line through to the ECM and the speedometer.

- What are the terminal numbers of the grounds at the ECM?

___________     ___________    ____________

- What is the terminal number of the ground at the speedometer?

___________

- Using an ORANGE marker trace the line from terminal CC-B at the ignition switch, through the ignition switch (in the on position) to terminal

1-CC-F of the ignition switch and continue on to terminal 3-B-A2 of the ECM.  Continue the ORANGE line from JT BK/YL to the start switch and the starter solenoid.

- Using a BLUE marker trace a line from terminal 3-B-H2 of the ECM to the low current side of the main relay.

- Using an ORANGE marker trace the line from the high current side of the main relay to the fuel pump, the ECM, the fuel injectors, the ignition coil, the speedometer, the tail lights, the low current side of the accessories relay, the low current side of the fan relay, the brake light switch, the brake relay and terminal HIBM-6.

- Imagine you have just closed the brake switch. Using a YELLOW marker trace a line from the brake switch to the brake lights.  Answer the following questions.

What is strange about this circuit? _____________________

Why is this done?_______________________________

- Using a BLUE marker  trace a line from terminal 3-B-G1 of the ECM to the Accessories (R3) relay and to the lights relay.

- Refer to the POWER DISTRIBUTION diagram.  What three items do the accessories (R3) relay supply power through the 5 amp # F7 fuse to?

___________________   ___________________    _________________

- On the general system schematic and using a BROWN marker trace a line from the accessories relay to 2WD/4WD switch, the winch switch and the switches illuminated power.

- With a BLUE marker circle the ETA electronic throttle TPS and the throttle accelerator sensor.

- Look directly below the TAS on the schematic for the 7 female terminals.  Using a green marker place a dot on each of the terminals.  What are these terminals used for ______________________________

- Identify the following in regards to the MAPTS.

  - Sensor ground = terminal ____________________

  - Pressure sensor power + terminal___________________

  - Temperature signal = terminal ___________________

  - Pressure signal = terminal ______________________

- A # 12 AWG (American Wire Gauge) is being used.  What is the max current, max fuse current and maximum wattage allowed on this wire?

max current = ______________

max fuse current = _______________

maximum wattage = ______________

- Identify the following symbols.

CE supplement diagram.

- Using a BLUE marker circle the high beam switch.

- What zone is the connector 1 - HIBM located?

Zone = __________________

- Referring to the TSS 2011 (1000) CE system schematic.  What component is the flasher system connected to?

________________________________

- Using a PURPLE marker and with the aid of the instructor circle the following connectors on the general system schematic.  1 HIBM 1,            1 HIBM 2, 1 HIBM 3, 1 HIBM 4, 1 HIBM 5, 1 HIBM 6 and 5 HIC 5-D.

- Referring to both schematics answer the following.

2 HIBM 1 of the CE schematic = ___________________

2 HIBM 2 of the CE schematic = power from the ____________relay to the ___________________switch

2 HIBM 3 of the CE schematic = power from the _____________ switch to the  ______________ beams

2 HIBM 4 of the CE schematic = power to the ______________ unit and the ___________ switch from fuse # ____________________

2 HIBM 5 of the CE schematic = signal from the ____________ unit to the ___________________ when the __________ beams are _______.

2 HIBM 6 of the CE schematic = power to the ________switch, the ____________ light, the _____________ switch and the __________unit

5 HIC 5 connector is where the ___________ __________ light is connected and it is located in the ____________ of the vehicle.

- Referring to the CE schematic what is diode D3 and D4 used for?

___________________________________________________________