| Unit |  | Task |  |
| --- | --- | --- | --- |
| 10 |  | 9 |  |
| Outlander 800 Max LTD |  |  |  |
|  |  | ATV | Side-by-Side |

# Fuel Pressure Testing

## Task Objective

At the completion of this task the technician will be able to properly perform a fuel pressure test on a multi-port fuel injected engine.  In addition he will be able to explain what the fuel pressure should be.

# Materials

Fuel pressure gauge

BUDS

Task sheet

Station binder and corresponding Outlander shop manual.

Repair order information

The customer states that the engine runs very rough and lack power cold or hot.  Says it seems as if it is running on one cylinder.  Further diagnosis has shown good compression test.  Engine has spark and injector pulse.

##### Introduction

The first step in performing a fuel pressure test is to relieve the pressure in the fuel rail.  The fuel pressure is relieved by activating the fuel pressure relief function on the activation screen in BUDS.

Warning:  When working on a pressurized fuel system, remember that the fuel lines remain under pressure at all times.  Depressurize the system before proceeding to disassemble.  Always proceed with care and use appropriate safety equipment when working on pressurized fuel systems.  Wear safety glasses and work in a well ventilated area.  Do not allow fuel to spill on hot engine parts and/or on electrical connectors.  Proceed with care when removing/installing high pressure test equipment or disconnecting fuel line connections.  Clean up any fuel spills that may occur.

Warning:  Ensure to verify fuel line connections are not damaged and that NO fuel line is disconnected prior to activating the electrical system or installing the safety lanyard on the DESS post.  When performing pressure tests ensure any pressure test equipment is installed before activating the electrical system or connecting the safety lanyard.  The fuel pump is activated each time the safety lanyard is installed and it builds pressure very quickly.

Fuel pressure testing is an important FIRST step testing procedure in fuel injection engines.  It is as important as a compression test as a preliminary test and low fuel pressure cannot be detected by the ECM.

# Multi Port Type Fuel Pressure Testing

Refer to the task below for the procedure and the proper Station Binder section for the correct fuel pressure specification.

# Procedures

|  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| - Follow the following steps and record the information below. |  |  |  |  |  |
| - What is the specification for the fuel pressure? – |  |  |  |  |  |
| - Relieve the fuel pressure using BUDS. |  |  |  |  |  |
| - Observe the fuel pressure gauge. Note: For the purposes of this training the fuel pressure gauge is already installed on the station for you. |  |  |  |  |  |
| - Activate the fuel pump with BUDS. |  |  |  |  |  |
| - Observe the fuel pressure gauge. |  |  |  |  |  |
| - Record the fuel pressure value recorded here. – |  |  |  |  |  |
| - Is the fuel pressure within specification? |  |  |  |  |  |
| - YES |  | NO |  |  |  |
| - Start the engine and record the fuel pressure. |  |  |  |  |  |
| - Record the fuel pressure value recorded here. – |  |  |  |  |  |
| - Is the fuel pressure within specification? |  |  |  |  |  |
| - YES |  | NO |  |  |  |

### Questions

1.  What have you learned from this task?

___________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

2.  List three causes of low fuel pressure.

- ___________________________________________

- ___________________________________________

- ___________________________________________

3.  Briefly describe the effect of low fuel pressure on an engine.

____________________________________________________________________________________________________________________________________________________________________________________________________________________________

4.  Briefly describe the effect of high fuel pressure on an engine.

____________________________________________________________________________________________________________________________________________________________________________________________________________________________

### Instructor sign off--  Go __________

| Unit |  | Task |  |
| --- | --- | --- | --- |
| 10 |  | 10 |  |
| Outlander 800 Max LTD |  |  |  |
|  |  | ATV | Side-by-Side |

# Fuel Injector Balance Test

## Task Objective

At the completion of this task the technician will be able to properly perform an injector balance test.  Further skills will be demonstrated by explaining why a balance test would be necessary.

# Materials

Fuel pressure gauge

BUDS

Task sheet

Station binder and corresponding Shop Manual

Repair Order Information

The customer states that the engine runs very rough and lacks power cold or hot.  Says it seems as if it is running on one cylinder.  Further diagnosis has shown good compression tests.  The engine has spark and injector pulse.

##### Introduction

The first step in performing a fuel pressure test is to relieve the pressure in the fuel rail.  The fuel pressure is relieved by activating the fuel pressure relief function with buds.

Another test that can be performed on any multiple injector system while the fuel pressure gauge is installed on the vehicle is called an injector balance test.  For the injector balance test BUDS can be used to activate the injectors for a predetermined amount of pulses.  First the fuel pump is activated using BUDS and the fuel pressure is checked.  With the gauge at full pressure, one injector is activated with BUDS and the pressure it drops to, is recorded. Then the fuel pump is activated again.  The next injector is activated and the pressure it drops to is, again recorded.  This sequence is repeated until all injectors have been activated and recorded.  All the injectors should deliver the same amount of fuel.  We say that they are all balanced.  All the readings should be within 10 KPA or (1.4 PSI) of each other.  If the reading of for one injector is off by more than that, the pintle inside the injector could be sticking or the injector screen could be clogged with debris.

# Fuel Injector Balance Test

Refer to the task list for the correct fuel pressure balance testing procedures.

# Procedures

|  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| - Follow the following steps and record the information below. |  |  |  |  |  |
| - What is the specification for the fuel pressure? – |  |  |  |  |  |
| - Activate the fuel pump with BUDS. |  |  |  |  |  |
| - Record the fuel pressure value recorded here. – |  |  |  |  |  |
| - Is the fuel pressure within specification? |  |  |  |  |  |
| - YES |  | NO |  |  |  |
| - Activate one fuel injector with BUDS. |  |  |  |  |  |
| - Record the value the fuel pressure dropped to here. – |  |  |  |  |  |
| - Activate the fuel pump with BUDS. |  |  |  |  |  |
| - Activate the other fuel injector with BUDS. |  |  |  |  |  |
| - Record the value the fuel pressure dropped to here. – |  |  |  |  |  |
| - Record the difference of the fuel pressure drop here. – |  |  |  |  |  |
| - Is the fuel pressure drop of both injectors within specification? |  |  |  |  |  |
| - YES |  | NO |  |  |  |

### Questions

1.  What have you learned from this task?

____________________________________________________________________________________________________________________________________________________________________________________________________________________________

2.  What would be indicated if the fuel pressure drop difference of one injector was more than 10 KPA (1.4 PSI)?

3.  What would be indicated if the fuel pressure drop difference of one injector was less than 10 KPA (1.4 PSI)?

### ______________________________________________________________________________________________________________

### Instructor sign off--  Go __________

### ______________________________________________________________________________________________________________

### ______________________________________________________________________________________________________________

### ______________________________________________________________________________________________________________

### ______________________________________________________________________________________________________________

### ______________________________________________________________________________________________________________

### ______________________________________________________________________________________________________________

### ______________________________________________________________________________________________________________

### ______________________________________________________________________________________________________________

### ______________________________________________________________________________________________________________

### ______________________________________________________________________________________________________________

### ______________________________________________________________________________________________________________

### ______________________________________________________________________________________________________________

### ______________________________________________________________________________________________________________

### ______________________________________________________________________________________________________________