| Static and dynamic tests Station | Task |
| --- | --- |
| 30 | 1 |
| SENSORS |  |

TASK OBJECTIVE

The objective of this station is to understand how the different types of sensors work and be familiar with them. The technician will be able to identify the sensor type, the voltage applied to it and the different tests to determinate the sensor is faulty or not.

At the same time he will be able to test the DESS post and the main relay.

INTRODUCTION

Generally speaking, all the inputs to and ECU (or any other vehicle module) are read by a couple of internal voltmeters.

These internal voltmeters can read:

- A voltage that can be interrupted by a switch ON/OFF

  - Starter switch, OPS

- A voltage generated by the sensor

  - CPS, Knock sensor, Oxygen sensor

- A voltage sent by the ECU itself that drops due to the increase of a resistance

  - Fuel lever sensor, CTS, TPS, MAPS

- A voltage that change from one value to another

  - TOPS, OTAS, CAPS, DIODE switch

- A voltage that changes progressively.

  - CTS, TPS, MAPS, TAS

Every sensor has its own method to vary the voltage:

- Hall effect switch

  - Activated by a magnet

  - Activate by the presence of a metal

- Hall effect sensor

- Voltage divisor

- Variable resistor

  - Activated mechanically

  - Activated by temperature

  - Activated by pressure (Piezo resistive)

- Generator

  - Generated by a magnet

  - Generated by a piezo-electric material

PROCEDURES

Before to start: Connect battery

Adjust the PWM to 5 volts

Don’t leave any tool on the 3 power lines on the center to avoid short cuts Check the base resistance of the multimeter leads and adjust it to 0

If the multimeter has no the reset option, reduce that value from every further resistance measurement.

There is no need to remove the cap of the test light

Never use the needle of the test light to puncture any wires

The voltage readings you get on this station can vary from a real unit where the sensors are connected to the ECU

Always use dielectric grease when inserting the backprobes on the connector wire seals.

#1	DESS POST

INTRODUCTION

Once the DESS cap is placed upon the DESS post, the micro switch within the post will close. Upon closing, the black/yellow wire which comes from the MPEM /ECU, it will complete its path to ground to activate and wake up the MPEM/ECU. As soon as the MPEM/ECU wakes up a signal is fed through the white/gray wire to the DESS post and through the chip in the DESS cap and finally to the black wire which goes back to the MPEM/ECU.  This is how the MPEM/ECU reads the chip in the cap to determine if the cap is correct or not, which if correct will allow the engine to be started.

PROCEDURES

Follow the steps and answer the questions below:

| 1. Check the continuity between D and the DESS outer ring |  |
| --- | --- |
| 2. The resistance should be less than 0.5 ohms Correct | Incorrect |
| 3. Check the continuity between C and the center pin of the DESS |  |
| 4. The resistance should be less than 0.5 ohms Correct | Incorrect |
| 5. Check the continuity between D and B |  |
| 6. It should be an open circuit: O.L. Correct | Incorrect |
| 7. Check the continuity between D and A: |  |
| 8. It should be an open circuit: O.L. Correct | Incorrect |
| 9. Check the continuity between A and B: |  |
| 10. It should be an open circuit: O.L. Correct | Incorrect |
| 11. Install the test light between 12+ and the ring |  |
| 12. Does it light bright? YES NO |  |
| 13. Install the magnet on top of the DESS |  |
| 14. Check the continuity between D and A |  |
| 15. The resistance should be less than 0.5 ohms Correct | Incorrect |
| 16. Check the continuity between D and B |  |
| 17. The resistance should be less than 0.5 ohms Correct | Incorrect |
| 18. Check the continuity between D and C |  |
| 19. It should be an open circuit: O.L. Correct | Incorrect |
| 20. Install the test light between 12V+ and A |  |
| 21. Does it light? YES NO |  |

22. Install the test light between 12V+ and B

23. Does it light? YES NO

24. Is the DESS post activation side good? YES NO

25. NOTE. Don’t use the test light if the DESS is connected to the ECU. The DESS is grounded trough the ECU and the amperage could be too high for the internal components of the ECU.

26. Measure the resistance of the lanyard between the internal contact:

27. NOTE. It has polarity. If no reading, change the polarity of the multimeter leads and try again. Increase the ohm scale if needed

28. Put the lanyard key on the DESS post

29. Check continuity between A and D:

30. Check continuity between B and D:

31. Is the magnet inside the DESS key good? YES NO

32. Check the resistance between C and D:

33. NOTE. It has polarity. If no reading, change the polarity of the. Increase the ohm scale if needed

34. Is the white /ground circuit good? YES NO

35. Remove the lanyard

#2	MAIN RELAY

INTRODUCTION

A relay is an electric switch that allows a low voltage or a low amperage circuit to control a high voltage or high amperage circuit.

The use of relays reduces the amount of heavy duty components needed to supply enough current to the entire electrical system of the unit.

As an example, if the main relay was not use, the DESS post should be able to hold all the current arriving to the modules, injectors, fuel pump ignition coils…

This would require both heavy duty DESS post and heavy gauge wires on the steering harness which add cost and complexity to the manufacturing o the vehicle.

Heavy duty components are difficult to locate on specific places like the handle bars or instrument panels

The relays have two parallel circuits: Low current circuit and high current circuit. The low current circuit is for the activation of the relay itself.

The high current circuit is the contact that opens or closes the high current flow to the specific component/s once activated by the low current circuit.

The relays can be normally open or normally closed and also can have a fifth terminal to have both options of the same relay.

PROCEDURES

Testing the high current circuit:

36. Check the resistance between 30 and 87:

37. Check resistance between 30 and 87A:

38. Check resistance between 87 and 87A:

39. Look at the drawing on top of the relay

40. Are the above results according to the drawing? YES NO

Testing the low current circuit:

41. Check the resistance of the electromagnet’s coil between 86 and 85

42. Install a jumper wire between 85 and A (DESS Post)

43. Install the DESS key on the DESS post

44. Is the relay activated? (click) YES NO

45. Check again the resistance between 30 and 87:

46. Check the resistance between 30 and 87A:

47. Check the resistance between 87 and 87A:

48. What has changed from the previous checks?

49. Check voltage between 87 and D:

50. This reading only shows you have voltage (potential) on the relay output.

51. Place the test light between 87 and D

52. Is it lighting? YES NO

53. This test only shows the relay allows enough current flow to power the light

54. With the help of another jumper wire keep the test light connected to 87 and D

55. Perform a voltage drop test between 30 and 87

56. Are the high current contacts inside the relay good? YES NO

57. This last test confirms the test light receives all the current it demands and is lighting at his full capacity.

58. Remove lanyard, jumper wires and test light.

#3	CPS

INTRODUCTION

The CPS is a small AC generator.

The signal from this sensor (AC voltage) is used by the ECU to determine the engine speed and the piston position in order to adjust the timing for the ignition and injection.

At the same time the ECU knows if the engine is accelerating or decelerating.

As any other AC generator, the CPS consist in a coil a magnetic field and a metal piece. The AC voltage is generated when any of the three components moves.

On the CPS the permanent magnet and the coil are static. The moving component is the metal piece (flywheel).

Every teeth of the flywheel attracts the magnetic field when passes in front of the CPS. The magnetic field tries to follow the teeth. When the teeth has passed the magnetic field returns to his original position

This movement of the magnetic field cuts the coil and induces a voltage on it. It is an AC voltage because the magnetic field is moved on both directions.

PROCEDURES

59. Check the resistance of the coil between 30 and 87:

60. This value can be checked on the specific shop manual

61. Prepare the multimeter to read AC voltage

62. Press the switch SW for 8 seconds just to get some RPM

63. Check the voltage between E and F:

64. This value can be confirmed with the specific shop manual

65. Stop the flywheel and place the diode between E and F

66. Press the switch SW again for 8 seconds

67. Is the diode lighting? YES NO

#4  FUEL LEVER SENSOR

INTRODUCTION

The fuel lever sensor is a variable resistor moved mechanically.

It is used to determinate the position of the fuel lever buoy inside the tank.

Being a variable resistor it creates a variable voltage drop on the wire/s connected to the ECU (or other module).

This voltage drop is then interpreted by the ECU, the cluster or directly by a fuel gauge.

PROCEDURES

68. With the help of two jumper wires check the resistance between I and J:

69. Lift the lever while slowly while checking the resistance

70. What is the resistance doing?

71. Install the test light between I and L

72. Move the lever slowly and observe the light

73. What is the light showing? A resistance A voltage drop Continuity from 12V+ to ground

#5	CTS

INTRODUCTION

The CTS is a variable resistor also.

In this case the resistance varies with the temperature (like on the MATS or the EGTS There are two types of temperature sensors: PTC (Positive Temperature Coefficient) and NTC (Negative Temperature Coefficient)

On the PTCs the resistance increases when the temperature increases On the NTC the resistance decreases when the

Being a variable resistor it creates a variable voltage drop on the wires connected to the ECU This voltage drop is interpreted by the ECU to adjust injection parameters.

The information from this sensor can be use to activate a fan cooler, to activated an overheating alarm or to activate the limp home mode.

Similar sensors are used to show air or lake temperatures on clusters.

PROCEDURES

74. Check resistance between G and H at room temperature:

75. Grab the sensor with your hand during a few seconds

76. What is the resistance doing?

77. What kind of temperature sensor is this? NTC PTC

78. The shop manuals have a table showing the resistance values at specific temperatures.

#6	TPS

INTRODUCTION

The TPS is a voltage divisor.

This type of sensor is used to indicate the position of a mechanical device.

They are used to indicate the throttle position, the suspension position, the gear box selection…

A voltage divisor works like a variable resistor.

Unlike a variable resistor, the voltage divisor uses three wires.

Two of the wires receive a reference voltage from the ECU (or other module).

The third wire is the signal wire which is mechanically moved across the internal resistance. The fact it uses 3 wires, there are two different variable voltage drops.

While in one side the voltage drop increases on the other side the voltage drop is reduced.

PROCEDURES

79. Note: In this board the TPS sensor is powered by 12 volts like on the iS suspension sensor

80. Check the reference voltage at TPS connector between 2 and 3

81. Use the two flat terminals to avoid damage on the female terminals of the connector

82. What is the voltage at the connector?

83. Install the test light between 2 and 3

84. Is it lighting bright? YES NO

85. Note: As said above this sensor can be used as a throttle position sensor. In that case it is powered by 5 volts sent by the ECU. Never connect the test light on that case. The amperage can be too high for the internal components of the ECU (there is no fuse between the sensor and the ECU)

86. Check the continuity of the signal wire between 1 and I (ECU)

87. Check the sensor resistance between 2 and 3 using the 3 wires connector from the tool box.

88. Turn the sensor knob clockwise and counterclockwise.

89. Is the resistance moving? YES NO

90. What does it mean?

91. Check the resistance between 1 and 3 while turning the knob slowly from the left end to the right end

92. Is the resistance increasing or decreasing?

93. Connect the leads between 1 and 3 and repeat the test

94. Is the resistance increasing or decreasing

95. Remove the 3 wire probe connector and connect and plug the power connector

96. Check voltage between K and L

97. Turn the knob to the left while checking the voltage between L and ECU (use a jumper wire to have one hand free

98. What is happening with the voltage?

99. Turn the knob to the left while checking the voltage

100. What is happening with the voltage?

101. Repeat the test between K and ECU and note the differences.

#7	DIODE SWITCH

INTRODUCTION

Unlike the sensors tested above the diode switch uses diodes to create different voltage drops. One diode creates a drop of approximately 0.5 volts

Two diodes in series will drop approximately 1 volt and so on. Inside the diode switch there are 8 diodes distributed in two rows.

They can be bypassed with the 4 switches thus creating different voltage drops that are interpreted by the cluster.

PROCEDURES

To be able to test this switch with a multimeter only it should be equipped with the diode test option + an internal resistor.

This internal resistor creates a voltage drop which is the reference for the added drops created by the diode switch.

The voltage and the current should be enough to cross the 4 diodes of every half of the diode switch.

Many multimeters don’t have the diode option. Others have it but they only can read one diode.

In those cases there is another method to test the Diode switch:

Using external 9v battery and resistor

Voltage shown varies according to resistance used and charge state of battery, however the percentage change when a switch button is pressed stays the same, for example –

|  | 33 ohms | 220 ohms | 560 ohms |
| --- | --- | --- | --- |
| Base voltage | 3.58 | 3.06 | 2.86 |
| Up Button | 1.92 (54%) | 1.57 (51%) | 1.46 (51%) |
| Down button | 2.77 (77%) | 2.32 (76%) | 2.17 (76%) |

Proposed test procedure –

- Use 9V battery and resister of 33 to 500 ohms

- Note base voltage,

- Press UP button, reading should be 50 - 55 % of baseline voltage

- Press DOWN button, reading should be 75 - 80 % of baseline voltage.

In our board we are going to use the battery voltage and the TPS to create a voltage drop down to 3V.

102. Adjust the TPS to obtain close to 3 volts between ECU and K

103. Install a Jumper wire between ECU and IS

104. Check the base voltage between IS and L and adjust it to 2.86v

105. Press IS up and check the voltage between IS and L:

106. Press IS down and check the voltage between IS and L:

107. Change the jumper wire to VTS

108. Press VTS up and check the voltage between IS and L:

109. Press VTS down and check the voltage between IS and L:

110. Are the results like on the 9V battery test? YES NO

As a clarification of the above test check the voltage drop created between the row of diodes

111. Install the jumper wire between ECU and D1

112. Adjust the base voltage between D1 and L to 2 volts

- Check the voltage drop between: - D1 and D2: - D2 and D3: - D1 and D4: - D1 and D5:

118. How many volts drop on every diode?

119. What is the total drop of the 4 diodes?

120. Place the another jumper wire between D2 and D4

121. What is the total drop now?

122. Why?

123. This is exactly what happens inside the diode switch when you press any of the buttons.

#8	KNOCK SENSOR

INTRODUCTION

The knock sensor is a piezo electric generator. Inside is a pressure sensitive crystal.

As pressure and vibration is applied to the crystal, a small AC voltage is generated and sent to the ECU. The ECU is programmed to detect the precise vibration frequency that fuel detonation produces. Only when these vibrations are sensed will the ECM respond with timing and mixture adjustments to eliminate detonation and avoid damage on the engine. When the engine is detonating no fault code is displayed since the sensor is doing his job.  The user can notice a reduction on the top rpm (+/- 400) until the detonation stops and the ECM goes back to the normal parameters.

A fault code related to the Knock sensor (P0325) indicates a failure on the sensor or its wiring.

Being the KS signal a very low voltage, the wires are protected by a special cover like the CPS and the wires are twisted between them on the visible portion, to avoid interferences. The proper bolt torque on this sensor is a must as it can affect to the internal pressure/vibrations.

PROCEDURES

124. Adjust the multimeter to 20 Mohm

125. Check for 5 Mohms between N and O

126. Is the resistance correct? YES NO

127. Place the LED between N and O and hit with the punch the metal part close to the sensor

128. Don't hit the sensor nor the bolt!!!

129. The LED should flash with every hit indicating the sensor is able to generate voltage. (Look closely since the light is weak)

130. Note: This test doesn’t prove the sensor will work properly above 5000 rpm.

#9	CAPS

INTRODUCTION

In order to determine the TDC on the compression stroke on a 4 stroke engine is necessary to install a position sensor on the camshaft since the CPS will send two signals per cycle (one during compression and another during valve overlap.

Other systems determine the TDC on compression based on the speed of the piston between the different strokes.

HALL EFFECT SWITCH

The Hall effect switch operates on the ‘Hall effect’, which is named after its discoverer Edwin Herbert Hall (1855-1938).

The Hall effect states in part that:

If a magnetic field is applied perpendicularly to the conductor, a voltage is generated transversely to the current flow direction in an electrical conductor (the Hall voltage)

The conductor or the “Hall element” is a small platelet made of semi-conductive material. Inside a Hall switch, the small voltage generated is used to switch a source voltage on and off to produce a digital output signal.

The Hall effect switch is a very accurate method by which to determine the exact position, or speed of rotation, of a spinning shaft. (Camshaft, crankshaft, driveshaft…)

The design of the Hall switch can vary, although most utilize a shutter or protrusions which pass by the sensor.

The shutter interrupts the magnetic field across from the semi-conductive electronic switch. As the shutter interrupts the magnetic field, the voltage output is turned on and off.

Hall effect sensors use three wires. One wire carries the positive voltage and a second wire supplies the sensor ground.

The third wire is the “toggle”. This wire carries the sensor’s output to the module. The voltage rises usually to the supply voltage, and falls close to zero depending on the position of the shutter.

The signal is a square wave and need no analog- to digital conversion to be read by the module.

PROCEDURES

131. Disconnect the sensor

132. Check for battery voltage at the connector between 1 and 3: YES NO

133. Connect the test light between 3 and ground

134. Is it lighting bright? YES NO

135. Note: Do not use the pin 1 as a ground since it is grounded by the ECU. The amperage of the test light could be too high for the internal components of the ECU

136. Check the continuity of the signal wire between 2 and ECU

A Hall effect sensor can’t be measured internally unlike a voltage divisor (TPS) so there are no static tests for it.

137. Connect the sensor

138. Check the voltage between ECU and P

139. Place a metallic object in front of the sensor

140. Is the voltage switching? YES NO

On a unit we can’t access to the signal wire at the ECU because if we remove the ECU connector the sensor losses its ground thus the sensor doesn’t work.

To check a Hall sensor on a unit it should be backprobed (or a “T” harness)

141. Install two back probes on the pins 2 and 3

- Note: Use some dielectric grease on the probes to avoid damage on the wire seals - Note: To be sure the backprobe is contacting with the pin remove the connector and check the continuity of the probe with the female terminal at the front of the connector.

144. Place a metallic object in front of the sensor

145. Do you obtain the same readings? YES NO

146. Note: If the sensor is checked while the engine is running it will read around 6 volts because the sensor switches from 0V to 12V faster than the multimeter is able to read.

#10	MAPS

INTRODUCTION

The MAP sensor monitors air pressure in the intake manifold or air box. The information they provide to the ECU is used to determine the engine load. In addition, when the electrical system is first activated, the information from the sensor is used to determine the altitude at which the machine is being operated and the correct fuel delivery parameters are selected for initial startup.

The MAPS has an internal silicon wafer that varies its resistance when is deformed by vacuum or pressure. An integrated circuit converts the fluctuations in resistance to a voltage that is sent to the ECU

It works with only 5 volts provided by the ECU so never use a 12V test light to check the reference voltage at the MAPS connector.

PROCEDURES

147. Disconnect the sensor

148. Using the flat terminal proves to avoid damage on the female pins, check the voltage between 1 and 2

149. Is the voltage correct? YES NO

150. Check the continuity of the signal wire between 3 and ECU

- Note: Due to the internal circuitry the sensor can’t be tested statically - Note: The 3 wires of the sensor are connected to the ECU so in order to test it dynamically it have to be backprobed

153. Connect the sensor and install the backprobes between 1 and 3

154. Note the voltage:

155. Note: This is an intermediate value meaning this sensor can work under vacuum and pressure. In fact this MAPS belongs to a supercharged engine. A MAPS used on an atmospheric engine only works in one direction and the starting voltage will be close to 0V.

156. With the manual pressure/vacuum pump apply a bit of vacuum on the sensor fitting

157. What is the voltage doing?

158. Now apply some pressure

- What is the voltage doing?

- Remove the backprobes and the pump

There is no pressure/voltage values table on the shop manuals but this is easy to get from a good known sensor.

On BUDS a good sensor should indicate the pressure as follows:

  - Engine stopped at sea level: 100 KPa

  - Engine running at idle: Between 40 and 60 KPa (Vacuum)

  - On a supercharged engine the pressure should increase as long the rpm increase

#11	OPS

INTRODUCTION

The Oil Pressure Switch provides a ground to the ECU when there is no oil pressure. Starting on MY2006 engines the OPS is normally closed meaning when the engine is not

running the switch is closed “sending” a ground signal to the ECU. When the engine is running the oil pressure opens the circuit.

If the engine is running and the oil pressure drops between 26-32 PSI, the switch closes and the ECU receives the ground signal to activate the alarm and to limit the rpm to avoid damage on the engine.

MY2005 and prior engines have a different logic and the oil switches are normally open so they aren’t interchangeable.

PROCEDURES

161. Check the switch is properly grounded to the engine by measuring the resistance between the metallic body of the sensor and the ground P (engine)

162. Is there a good ground?

- Check the continuity of the signal wire from the ECU to the switch connector - Note: Use the flat blue terminal to avoid damage on the connector pin

165. Is the wire good? YES NO

166. Reconnect the sensor

167. Check the continuity between ECU and P (engine ground)

168. Is the switch open or closed?

169. Place the test light between 30 and the OPS signal wire (ECU)

170. Does it light? YES NO

171. Note this test will not damage the ECU on the vehicle since it will be disconnected and the ECM adapter 420 277 010 installed on the connector

172. With the help of a jumper wire keep the test light grounded through the signal wire (ECU)

173. Install the pressure pump on the switch and apply some pressure

174. At what pressure the test light stops lighting?

175. Is that pressure correct?

176. Remove the pump and the test light.

#12  OTAS

INTRODUCTION

The OTAS switch (Off Throttle Assistance Steering) is a Hall effect switch that can be activated by one of the two magnets properly positioned on the steering column to indicate a full turn to the right or to the left.

Like the CAPS, the reference voltage is the battery voltage.

PROCEDURES

177. Disconnect the sensor and check for battery voltage between A and B

178. Is it correct? YES NO

179. Check the continuity of the signal wire from C to the ECU.

180. Is the wire good? YES NO

181. Place the test light between A and ground (P) Note: Don’t use the terminal B as a ground because on a real unit this wire is grounded by the ECU and the current drawn by the test light could overload the ECU internal circuitry.

- Is it lighting bright?  YES	NO

- Connect the sensor

Like the CAPS, the OTAS sensor only can be tested while it is connected to the ECU (or to an external power supply like in this exercise) so it have to be backprobed.

| 184. Check the voltage between B and C: |  |  |
| --- | --- | --- |
| 185. Move one of the magnets to the center of the sensor |  |  |
| 186. Note the voltage: |  |  |
| 187. Move the other magnet to the center of the sensor |  |  |
| 188. Note the voltage: |  |  |
| 189. Is the sensor detecting the straight position and the full turns? | YES | NO |

#13	TAS

INTRODUCTION

The throttle accelerator sensor (TAS) is a double Hall Effect sensor that sends a signal to the ECM which is proportional to the accelerator lever. The redundancy is used for security purposes. The voltage outputs of the Hall Effect sensors are different.

According to the torque demand from the TAS, the ECM powers the throttle body motor to rotate towards open or close the throttle plate. When the ECM detects through the TPS that the throttle plate has reached the targeted opening, the ECM stops the throttle actuator.

The fact the throttle lever isn’t linked by a mechanical throttle cable to the throttle body allows the ECM to move the throttle plate irrespectively from the TAS input in cases like idle control, limp home mode, touring mode, cruiser mode, rpm limiter, OTAS activation…

The TAS is powered and grounded by the ECU and due it is a double hall sensor it has two 5 volts inputs, 2 grounds and two output signal wires.

As any other hall sensor or switch it can’t be tested internally so it have to be connected and backprobed.  (It is also possible to test it with a “T” harness)

PROCEDURES

190. Disconnect the sensor and check for 5 volts between A and B: YES NO

191. Check for 5 volts between D and E: YES NO

192. Check the continuity of the signal wire C to ECU

193. Check the continuity of the signal wire F to ECU

194. Are the signal wires correct? YES NO

195. Connect the sensor

196. Lubricate the backprobes with the dielectric grease

197. Check the voltage between A and the signal wire C:

198. Move the lever slowly and observe the voltage

199. Is the voltage increasing or decreasing?

200. Check the voltage between the ground B and the signal wire C:

201. Move the lever slowly and observe the voltage

202. Is the voltage increasing or decreasing?

203. Check the voltage between D and the signal wire F:

204. Move the lever slowly and observe the voltage

205. Is the voltage increasing or decreasing?

206. Check the voltage between the ground wire E and the signal wire F:

207. Move the lever slowly and observe the voltage

208. Is the voltage increasing or decreasing?

The models equipped with IBR (Inteligent Brake and Reverse) use an additional lever on the left side of the handlebar: The BRLS (Brake and Reverse Lever Sensor)

It is also a double Hall effect switch and it works the same way but powered by the IBR

module.

#14	TOPS

INTRODUCTION

The purpose of the Tip Over Protection Switch is the engine protection in the event the watercraft tips over.

The TOPS is a Hall effect switch located under the blow-by valve. Inside the blow-bu valve there is a sliding pin with a magnet maintained by the gravity close to the Hall switch which is located just below.

The proximity of the magnet maintains the switch on the “off “position

If the unit is capsized the magnet separates from the switch by gravity so the switch sends a signal to the ECU who will shut off the engine.

PROCEDURES

209. Disconnect the sensor and check for 5 volts between 3 and 2: YES NO

210. Check the continuity of the signal wire 1 to ECU

211. Is it correct? YES NO

212. Connect the sensor

213. Check the voltage with the backprobes between 2 and 1:

214. Install the pin with the magnet on the sensor

215. Does the voltage vary? YES NO

Note: Hall Effect switches have no a real OFF position. There is always some output voltage that allows the ECU to verify the sensor is operational despite it is activated or not.

That is not possible on a normal switch due to the signal of an open switch is the same if the switch is disconnected or the wires cut.

Questions

  - What have you learned on this board?

  - Which sensors/switches work at 5 volts?

3.   Which sensors/switches work at 12 volts?

- Which sensor can work at 5 or 12 volts?

- Which sensors are voltage generators?

- The best way to check a wire is powering a load trough it but, when this method should never be done?

Instructor sign off-- Go